# Rigging Material Handling

## Inspection & Defective Removal

All [Company_Name] rigging equipment for material handling shall be inspected prior to use on each shift and as necessary during its use to ensure that it is safe.

[Company_Name] requires that all defective rigging equipment must be removed from service immediately.

Rigging equipment shall not be loaded in excess of its recommended safe working load, as prescribed in Tables H-1 through H-20 in 1926.252(e) for the specific equipment.

Load identification must be attached to the rigging before it is to be allowed to move.

Rigging equipment, when not in use, shall be removed from the immediate work area so as not to present a hazard to [Company_Name] employees.

Tag lines shall be used unless their use creates an unsafe condition.

## Hook Throat Opening

Hooks on overhaul ball assemblies, lower load blocks, or other attachment assemblies shall be of a type that can be closed and locked, eliminating the hook throat opening.

Alternatively, an alloy anchor type shackle with a bolt, nut and retaining pin may be used.

All employees shall be kept clear of loads about to be lifted and of suspended loads.