# Bin, Bunkers, Hoppers and Material Storage

## Bins, Bunkers, and Hoppers.

(a) Every open bin, bunker, hopper, and dangerous equipment whose upper edge is less than 36 inches above the floor or working level shall be equipped with a standard railing around its edges, or a grating or grille shall cover the top.

Where grille or grating is the only protection, it shall have no opening whose

least dimension exceeds 10 inches. Where railings are used they may be

temporarily removed to provide necessary working openings.

(b) The grating or grille shall be of strength sufficient to withstand any load

that is customarily imposed upon it.

(c) Where bins, bunkers, or hoppers are loaded by backing an automotive truck to one edge, there shall be installed bumper stops not less than 10 by 10 inches,

securely fastened in a manner to prevent the truck overrunning the runway. At

least 8- by 8-inch timbers or equivalent shall be securely fastened along the

sides of the runway to prevent a truck overrunning the sides of the runway.

(d) Open Top Bins.

(1) Where employees are permitted or required to work in or above open top bins

over 8 feet in depth, an upper working area consisting of a platform or walkway

shall be provided, and shall be guarded with a standard railing and toeboard.

(2) Personal fall arrest or restraint systems complying with the requirements shall be provided and used by employees when above requirements are not met.

## Piling Material.

(a) Piles of brick, tile, masonry blocks, and similar material shall be

stabilized against falling by the use of headers at least every sixth course.

When a loose brick stack reaches a height of 4 feet, it shall be tapered back at

least 2 inches in every foot of height above the 4-foot level. Brick stacks

shall not be more than 7 feet in height. When masonry blocks are stacked higher

than 6 feet, the stack shall be tapered back one-half block per tier above the

6-foot level.

(b) In piling bags of cement or other material more than 5 feet high, the face

of the pile shall be tapered back (except where supported by walls or

otherwise), or the sacks shall be so tied in horizontal layers as to prevent

them from falling or collapsing.

(c) Piles of lumber, structural steel, and similar materials shall be maintained

so that the stack is stable against falling or collapsing. Headers, crosspieces,

or other means shall be used as needed in the pile to prevent slipping, tipping,

or collapsing. Lumber piles shall not exceed 20 feet in height provided that

lumber to be handled manually shall not be stacked more than 16 feet high.

(d) Sheetrock, plywood, trusses, and similar material shall not be stacked on

edge unless positively secured against tipping or falling.

(e) Material inside buildings under construction shall not be placed within 6

feet of any hoistway or inside floor openings, nor within 10 feet of an exterior

wall which does not extend above the top of the material unless the material is

positively barricaded, placed, or secured to prevent it from falling.

(f) The maximum safe load limits of floors, or portions of floors, used for

storage areas within buildings and structures, shall not be exceeded.

(g) Materials which may cause a hazardous reaction or unstable condition while

in storage shall be segregated.