# Excavations & Trenching

## Excavations & Trenching

Scope and application. This article applies to all open excavations made in the earth's surface. Excavations are defined to include trenches.

Definitions applicable to this article.

Accepted engineering practices means those requirements which are compatible

with standards of practice required by a registered professional engineer.

Aluminum hydraulic shoring. A pre-engineered shoring system comprised of

aluminum hydraulic cylinders (crossbraces) used in conjunction with vertical

rails (uprights) or horizontal rails (walers). Such system is designed

specifically to support the sidewalls of an excavation and prevent cave-ins.

Bell-bottom pier hole. A type of shaft or footing excavation, the bottom of

which is made larger than the cross section above to form a belled shape.

Benching (Benching system). A method of protecting employees from cave-ins by excavating the sides of an excavation to form one or a series of horizontal

levels or steps, usually with vertical or near-vertical surfaces between levels.

Cave-in. The separation of a mass of soil or rock material from the side of an

excavation, or the loss of soil from under a trench shield or support system,

and its sudden movement into the excavation, either by falling or sliding, in

sufficient quantity so that it could entrap, bury, or otherwise injure and

immobilize a person.

Crossbraces: The horizontal members of a shoring system installed perpendicular to the sides of the excavation, the ends of which bear against either uprights or wales.

Excavation. Any man-made cut, cavity, trench, or depression in an earth surface,

formed by earth removal.

Faces or sides. The vertical or inclined earth surfaces formed as a result of

excavation work.

Failure. The breakage, displacement, or permanent deformation of a structural

member or connection so as to reduce its structural integrity and it’s supportive

capabilities.

Hazardous atmosphere. An atmosphere which by reason of being explosive,

flammable, poisonous, corrosive, oxidizing, irritating, oxygen deficient, toxic,

or otherwise harmful, may cause death, illness, or injury.

Kickout: The accidental release or failure of a cross brace.

Protective system. A method of protecting employees from cave-ins, from material that could fall or roll from an excavation face or into an excavation, or from the collapse of adjacent structures. Protective systems include support systems, sloping and benching systems, shield systems, and other systems that provide the necessary protection.

Ramp. An inclined walking or working surface that is used to gain access to one

point from another, and is constructed from earth or from structural materials

such as steel or wood.

Registered professional engineer. A person who is registered as a professional

engineer in the state where the work is to be performed. However, a professional

engineer, registered in any state is deemed to be a "registered professional

engineer" within the meaning of this standard when approving designs for

"manufactured protective systems" or "tabulated data" to be used in interstate

commerce.

Sheeting. The members of a shoring system that retain the earth in position and

in turn are supported by other members of the shoring system.

Shield (Shield system). A structure that is able to withstand the forces imposed

on it by a cave-in and thereby protect employees within the structure. Shields

can be permanent structures or can be designed to be portable and moved along as work progresses. Additionally, shields can be either pre-manufactured or

job-built in accordance with Section 1541.1(c)(3) or (c)(4). Shields used in

trenches are usually referred to as "trench boxes" or "trench shields."

Shoring (Shoring system). A structure such as a metal hydraulic, mechanical or

timber shoring system that supports the sides of an excavation and which is

designed to prevent cave-ins.

Sides. See "Faces."

Sloping (Sloping system). A method of protecting employees from cave-ins by

excavating to form sides of an excavation that are inclined away from the

excavation so as to prevent cave-ins. The angle of incline required to prevent a

cave-in varies with differences in such factors as the soil type, environmental

conditions of exposure, and application of surcharge loads. Stable rock. Natural solid mineral material that can be excavated with vertical sides and will remain intact while exposed. Unstable rock is considered to be stable when the rock material on the side or sides of the excavation is secured against caving-in or movement by rock bolts or by another protective system that has been designed by a registered professional engineer.

Structural ramp. A ramp built of steel or wood, usually used for vehicle access.

Ramps made of soil or rocks are not considered structural ramps.

Support system. A structure such as underpinning, bracing, or shoring, which

provides support to an adjacent structure, underground installation, or the

sides of an excavation.

Tabulated data. Tables and charts approved by a registered professional engineer and used to design and construct a protective system.

Trench (Trench excavation). A narrow excavation (in relation to its length) made

below the surface of the ground. In general, the depth is greater than the

width, but the width of a trench (measured at the bottom) is not greater than 15

feet. If forms or other structures are installed or constructed in an excavation

so as to reduce the dimension measured from the forms or structure to the side

of the excavation to 15 feet or less, (measured at the bottom of the

excavation), the excavation is also considered to be a trench.

Trench box. See "Shield."

Trench shield. See "Shield."

Uprights. The vertical members of a trench shoring system placed in contact with

the earth and usually positioned so that individual members do not contact each

other. Uprights placed so that individual members are closely spaced, in contact

with or interconnected to each other, are often called "sheeting."

Wales. Horizontal members of a shoring system placed parallel to the excavation

face whose sides bear against the vertical members of the shoring system or

earth.

## General Requirements.

(a) Surface encumbrances. All surface encumbrances that are located so as to

create a hazard to employees shall be removed or supported, as necessary, to

safeguard employees.

(b) Underground installations.

(1) The estimated location of utility installations, such as sewer, telephone,

fuel, electric, water lines, or any other underground installations that

reasonably may be expected to be encountered during excavation work, shall be

determined prior to opening an excavation.

(2) All Regional Notification Centers as defined by Government Code Section

4216(h) in the area involved and all known owners of underground facilities in

the area who are not members of a Notification Center shall be advised of the

proposed work at least 2 working days prior to the start of any digging or

excavation work. EXCEPTION: Emergency repair work to underground facilities.

(3) When excavation operations approach the estimated location of underground

installations, the exact location of the installations shall be determined by

safe and acceptable means.

(4) While the excavation is open, underground installations shall be protected,

supported, or removed as necessary to safeguard employees.

(c) Access and egress.

(1) Structural ramps.

(A) Structural ramps that are used solely by employees as a means of access or

egress from excavations shall be designed by a competent person. Structural

ramps used for access or egress of equipment shall be designed by a competent

person qualified in structural design, and shall be constructed in accordance

with the design.

(B) Ramps and runways constructed of two or more structural members shall have the structural members connected together to prevent displacement.

(C) Structural members used for ramps and runways shall be of uniform thickness.

(D) Cleats or other appropriate means used to connect runway structural members shall be attached to the bottom of the runway or shall be attached in a manner to prevent tripping.

(E) Structural ramps used in lieu of steps shall be provided with cleats or

other surface treatments to the top surface to prevent slipping.

(2) Means of egress from trench excavations.

A stairway, ladder, ramp or other safe means of egress shall be located in

trench excavations that are 4 feet or more in depth so as to require no more

than 25 feet of lateral travel for employees.

(d) Exposure to vehicular traffic. Employees exposed to public vehicular traffic

shall be provided with, and shall wear, warning vests or other suitable garments

marked with or made of reflectorized or high-visibility material.

(e) Exposure to falling loads. No employee shall be permitted underneath loads

handled by lifting or digging equipment. Employees shall be required to stand

away from any vehicle being loaded or unloaded to avoid being struck by any

spillage or falling materials. Operators may remain in the cabs of vehicles

being loaded or unloaded when the vehicles are equipped, in accordance with

Section 1591(e), to provide adequate protection for the operator during loading

and unloading operations.

(f) Warning system for mobile equipment. When mobile equipment is operated

adjacent to an excavation, or when such equipment is required to approach the

edge of an excavation, and the operator does not have a clear and direct view of

the edge of the excavation, a warning system shall be utilized such as

barricades, hand or mechanical signals, or stop logs. If possible, the grade

should be away from the excavation.

(g) Hazardous atmospheres.

(1) Testing and controls. In addition to the requirements set forth in the

Construction Safety Orders and the General Industry Safety Orders to prevent

exposure to harmful levels of atmospheric contaminants and to assure acceptable atmospheric conditions, the following requirements shall apply:

(A) Where oxygen deficiency (atmospheres containing less than 19.5 percent

oxygen) or a hazardous atmosphere exists or could reasonably be expected to

exist, such as in excavations in landfill areas or excavations in areas where

hazardous substances are stored nearby, the atmospheres in the excavation shall be tested before employees enter excavations greater than 4 feet in depth.

(B) Adequate precautions shall be taken to prevent employee exposure to

atmospheres containing less than 19.5 percent oxygen and other hazardous

atmospheres. These precautions include providing proper respiratory protection

or ventilation.

(C) Adequate precaution shall be taken such as providing ventilation, to prevent

employee exposure to an atmosphere containing a concentration of a flammable gas in excess of 20 percent of the lower flammable limit of the gas.

(D) When controls are used that are intended to reduce the level of atmospheric

contaminants to acceptable levels, testing shall be conducted as often as

necessary to ensure that the atmosphere remains safe.

(2) Emergency rescue equipment.

(A) Emergency rescue equipment, such as breathing apparatus, a safety harness

and line, or a basket stretcher, shall be readily available where hazardous

atmospheric conditions exist or may reasonably be expected to develop during

work in an excavation. This equipment shall be attended when in use.

(B) Employees entering bell-bottom pier holes, or other similar deep and

confined footing excavations, shall wear a harness with a lifeline securely

attached to it. The lifeline shall be separate from any line used to handle

materials, and shall be individually attended at all times while the employee

wearing the lifeline is in the excavation.

(h) Protection from hazards associated with water accumulation.

(1) Employees shall not work in excavations in which there is accumulated water,

or in excavations in which water is accumulating, unless adequate precautions

have been taken to protect employees against the hazards posed by water

accumulation. The precautions necessary to protect employees adequately vary

with each situation, but could include special support or shield systems to

protect from cave-ins, water removal to control the level of accumulating water,

or use of a safety harness and lifeline.

(2) If water is controlled or prevented from accumulating by the use of water

removal equipment, the water removal equipment and operations shall be monitored

by a competent person to ensure proper operation.

(3) If excavation work interrupts the natural drainage of surface water (such as

streams), diversion ditches, dikes, or other suitable means shall be used to

prevent surface water from entering the excavation and to provide adequate

drainage of the area adjacent to the excavation. Excavations subject to runoff

from heavy rains will require an inspection by a competent person and compliance with Sections (h)(1) and (h)(2).

(i) Stability of adjacent structures.

(1) Where the stability of adjoining buildings, walls, or other structures is

endangered by excavation operations, support systems such as shoring, bracing,

or underpinning shall be provided to ensure the stability of such structures for

the protection of employees.

(2) Excavation below the level of the base or footing of any foundation or

retaining wall that could be reasonably expected to pose a hazard to employees

shall not be permitted except when:

(A) A support system, such as underpinning, is provided to ensure the safety of

employees and the stability of the structure; or

(B) The excavation is in stable rock; or

(C) A registered professional engineer has approved the determination that such

excavation work will not pose a hazard to employees.

(3) Sidewalks, pavements and appurtenant structure shall not be undermined

unless a support system or another method of protection is provided to protect

employees from the possible collapse of such structures.

(j) Protection of employees from loose rock or soil.

(1) Adequate protection shall be provided to protect employees from loose rock

or soil that could pose a hazard by falling or rolling from an excavation face.

Such protection shall consist of scaling to remove loose material; installation

of protective barricades at intervals as necessary on the face to stop and

contain falling material; or other means that provide equivalent protection.

(2) Employees shall be protected from excavated or other materials or equipment

that could pose a hazard by falling or rolling into excavations. Protection

shall be provided by placing and keeping such materials or equipment at least 2 feet from the edge of excavations, or by the use of retaining devices that are

sufficient to prevent materials or equipment from falling or rolling into

excavations, or by a combination of both if necessary.

(k) Inspection.

(1) Daily inspections of excavations, the adjacent areas, and protective systems

shall be made by a competent person for evidence of a situation that could

result in possible cave-ins, indications of failure of protective systems,

hazardous atmospheres, or other hazardous conditions. An inspection shall be

conducted by the competent person prior to the start of work and as needed

throughout the shift. Inspections shall also be made after every rain storm or

other hazard increasing occurrence. These inspections are only required when

employee exposure can be reasonably anticipated.

(2) Where the competent person finds evidence of a situation that could result

in a possible cave-in, indications of failure of protective systems, hazardous

atmospheres, or other hazardous conditions, exposed employees shall be removed

from the hazardous area until the necessary precautions have been taken to

ensure their safety.

(l) Fall protection.

(1) Where employees or equipment are required or permitted to cross over

excavations over 6 feet and wider than 30 inches, walkways or bridges with

standard guardrails shall be provided.

(2) Adequate barrier physical protection shall be provided at all remotely

located excavations. All wells, pits, shafts, etc., shall be barricaded or

covered. Upon completion of exploration and other similar operations, temporary

wells, pits, shafts, etc., shall be backfilled.

## Requirements for Protective Systems.

(a) Protection of employees in excavations.

(1) Each employee in an excavation shall be protected from cave-ins by an

adequate protective system designed in accordance with Section (b) or (c) below

except when:

(A) Excavations are made entirely in stable rock; or

(B) Excavations are less than 5 feet in depth and examination of the ground by a

competent person provides no indication of a potential cave-in.

(2) Protective systems shall have the capacity to resist without failure all

loads that are intended or could reasonably be expected to be applied or

transmitted to the system.

(b) Design of sloping and benching systems. The slopes and configurations of

sloping and benching systems shall be selected and constructed by the company

or his designee and shall be in accordance with the requirements of Section

(b)(1), Section (b)(2), Section (b)(3), or Section (b)(4), as follows:

(1) Option (1) --Allowable configurations and slopes.

(A) Excavations shall be sloped at an angle not steeper than one and one-half

horizontal to one vertical (34 degrees measured from the horizontal), unless the

company uses one of the other options listed below.

(B) Slopes specified in Section 1541.1(b)(1)(A) shall be excavated to form

configurations that are in accordance with the slopes shown for Type C soil in

Appendix B to this article.

(2) Option (2) --Determination of slopes and configurations using Appendices A

and B. Maximum allowable slopes, and allowable configurations for sloping and

benching systems, shall be determined in accordance with the conditions and

requirements set forth in Appendices A and B to this article.

(3) Option (3) --Designs using other tabulated data.

(A) Designs of sloping or benching systems shall be selected from and be in

accordance with tabulated data, such as tables and charts.

(B) The tabulated data shall be in written form and shall include all of the

following:

1. Identification of the parameters that affect the selection of a sloping or

benching system drawn from such data;

2. Identification of the limits of use of the data, to include the magnitude and

configuration of slopes determined to be safe;

3. Explanatory information as may be necessary to aid the user in making a

correct selection of a protective system from the data.

4. At least one copy of the tabulated data which identifies the registered

professional engineer who approved the data, shall be maintained at the jobsite

during construction of the protective system. After that time the data may be

stored off the jobsite, but a copy of the data shall be made available to the

Division upon request.

(4) Option (4) --Design by a registered professional engineer.

(A) Sloping and benching systems not utilizing Option (1) or Option (2) or

Option (3) under Section 1541.1(b) shall be stamped and signed by a registered

professional engineer.

(B) Designs shall be in written form and shall include at least the following:

1. The magnitude of the slopes that were determined to be safe for the

particular project;

2. The configurations that were determined to be safe for the particular

project;

3. The identity of the registered professional engineer approving the design.

(C) At least one copy of the design shall be maintained at the jobsite while the

slope is being constructed. After that time the design need not be at the

jobsite, but a copy shall be made available to the Division upon request.

(c) Design of support systems, shield systems, and other protective systems.

Designs of support systems, shield systems, and other protective systems shall

be selected and constructed by the company or his designee and shall be in

accordance with the requirements of Section (c)(1); or, in the alternative, Section (c)(2); or, in the alternative, Section (c)(3); or, in the alternative, Section (c)(4) as follows:

(1) Option (1) --Designs using Appendices A, C and D. Designs for timber shoring in trenches shall be determined in accordance with the conditions and

requirements set forth in Appendices A and C to this article. Designs for

aluminum hydraulic shoring shall be in accordance with Section (c)(2), but

if manufacturer's tabulated data cannot be utilized, designs shall be in

accordance with Appendix D.

(2) Option (2) --Designs Using Manufacturer's Tabulated Data.

(A) Design of support systems, shield systems, or other protective systems that

are drawn from manufacturer's tabulated data shall be in accordance with all

specifications, recommendations, and limitations issued or made by the

manufacturer.

(B) Deviation from the specifications, recommendations, and limitations issued

or made by the manufacturer shall only be allowed after the manufacturer issues

specific written approval.

(C) Manufacturer's specifications, recommendations, and limitations, and

manufacturer's approval to deviate from the specifications, recommendations, and limitations shall be in written form at the jobsite during construction of the

protective system. After that time this data may be stored off the jobsite, but

a copy shall be made available to the Division upon request.

(3) Option (3) --Designs using other tabulated data.

(A) Designs of support systems, shield systems, or other protective systems

shall be selected from and be in accordance with tabulated data, such as tables

and charts.

(B) The tabulated data shall be in written form and include all of the following:

1. Identification of the parameters that affect the selection of a protective

system drawn from such data;

2. Identification of the limits of use of the data;

3. Explanatory information as may be necessary to aid the user in making a

correct selection of a protective system from the data.

(C) At least one copy of the tabulated data, which identifies the registered

professional engineer who approved the data, shall be maintained at the jobsite

during construction of the protective system. After that time the data may be

stored off the jobsite, but a copy of the data shall be made available to the

Division upon request.

(4) Option (4) --Design by a registered professional engineer.

(A) Support systems, shield systems, and other protective systems not utilizing

Option 1, Option 2, or Option 3, above, shall be approved by a registered

professional engineer.

(B) Designs shall be in written form and shall include the following:

1. A plan indicating the sizes, types, and configurations of the materials to be

used in the protective system; and

2. The identity of the registered professional engineer approving the design.

(C) At least one copy of the design shall be maintained at the jobsite during

construction of the protective system. After that time, the design may be stored

off the jobsite, but a copy of the design shall be made available to the

Division upon request.

(d) Materials and equipment.

(1) Materials and equipment used for protective systems shall be free from

damage or defects that might impair their proper function.

(2) Manufactured materials and equipment used for protective systems shall be

used and maintained in a manner that is consistent with the recommendations of

the manufacturer, and in a manner that will prevent employee exposure to

hazards.

(3) When material or equipment that is used for protective systems is damaged, a competent person shall examine the material or equipment and evaluate its

suitability for continued use. If the competent person cannot assure the

material or equipment is able to support the intended loads or is otherwise

suitable for safe use, then such material or equipment shall be removed from

service, and shall be evaluated and approved by a registered professional

engineer before being returned to service.

(e) Installation and removal of supports.

(1) General.

(A) Members of support systems shall be securely connected together to prevent

sliding, falling, kickouts, or other predictable failure.

(B) Support systems shall be installed and removed in a manner that protects

employees from cave-ins, structural collapses, or from being struck by members

of the support system.

(C) Individual members of support systems shall not be subjected to loads

exceeding those which those members were designed to withstand.

(D) Before temporary removal of individual members begins, additional

precautions shall be taken to ensure the safety of employees, such as installing

other structural members to carry the loads imposed on the support system.

(E) Removal shall begin at, and progress from, the bottom of the excavation.

Members shall be released slowly so as to note any indication of possible

failure of the remaining members of the structure or possible cave-in of the

sides of the excavation.

(F) Backfilling shall progress together with the removal of support systems from

excavations.

(2) Additional requirements for support systems for trench excavations.

(A) Excavation of material to a level no greater than 2 feet below the bottom of

the members of a support system shall be permitted, but only if the system is

designed to resist the forces calculated for the full depth of the trench, and

there are no indications while the trench is open of a possible loss of soil

from behind or below the bottom of the support system.

(B) Installation of a support system shall be closely coordinated with the

excavation of trenches.

(f) Sloping and benching systems.

Employees shall not be permitted to work on the faces of sloped or benched excavations at levels above other employees except when employees at the lower levels are adequately protected from the hazard of falling, rolling, or sliding material or equipment.

(g) Shield systems.

(1) General.

(A) Shield systems shall not be subjected to loads exceeding those which the

system was designed to withstand.

(B) Shields shall be installed in a manner to restrict lateral or other hazardous movement of the shield in the event of the application of sudden lateral loads.

(C) Employees shall be protected from the hazard of cave-ins when entering or

exiting the areas protected by shields.

(D) Employees shall not be allowed in shields when shields are being installed,

removed, or moved vertically.

(2) Additional requirements for shield systems used in trench excavations. The

sides of the shield shall extend a minimum of 18 inches above the vertical walls

of compound excavations . On vertically cut trenches, the shield shall extend to at least the catch point of the trench. Excavations of earth material to a level not greater than 2 feet below the bottom of a shield shall be permitted, but only if the shield is designed to resist the forces calculated for the full depth of the trench, and there are no indications while the trench is open of a possible loss of soil

from behind or below the bottom of the shield.

### Requirements for Protective Systems, Appendix A

Soil Classification

(a) Scope and application.

(1) Scope. This appendix describes a method of classifying soil and rock

deposits based on site and environmental conditions, and on the structure and

composition of the earth deposits. The appendix contains definitions, sets forth

requirements, and describes acceptable visual and manual tests for use in

classifying soils.

(2) Application. This appendix applies when a sloping or benching system is

designed in accordance with the requirements set forth in Section (b)(2)

as a method of protection for employees from cave-ins. This appendix also

applies when timber shoring for excavations is designed as a method of

protection from cave-ins in accordance with Appendix C to this article, and when

aluminum hydraulic shoring is designed in accordance with Appendix D. This

appendix also applies if other protective systems are designed and selected for

use from data prepared in accordance with the requirements set forth in Section

(c), and the use of the data is predicated on the use of the soil

classification system set forth in this appendix.

(b) Definitions.

Cemented soil. A soil in which the particles are held together by a chemical

agent, such a calcium carbonate, such that a hand-size sample cannot be crushed

into powder or individual soil particles by finger pressure.

Cohesive soil. Clay (fine grained soil), or soil with a high clay content, which

has cohesive strength. Cohesive soil does not crumble, can be excavated with

vertical side slopes, and is plastic when moist. Cohesive soil is hard to break

up when dry, and exhibits significant cohesion when submerged. Cohesive soils

include clayey silt, sandy clay, silty clay, clay and organic clay.

Dry soil. Soil that does not exhibit visible signs of moisture content.

Fissured. A soil material that has a tendency to break along definite planes of

fracture with little resistance, or a material that exhibits open cracks, such

as tension cracks, in an exposed surface.

Granular soil. Gravel, sand, or silt (coarse grained soil) with little or no

clay content. Granular soil has no cohesive strength. Some moist granular soils

exhibit apparent cohesion. Granular soil cannot be molded when moist and

crumbles easily when dry.

Layered system. Two or more distinctly different soil or rock types arranged in

layers. Micaceous seams or weakened planes in rock or shale are considered

layered.

Moist soil. A condition in which a soil looks and feels damp. Moist cohesive

soil can easily be shaped into a ball and rolled into small diameter threads

before crumbling. Moist granular soil that contains some cohesive material will

exhibit signs of cohesion between particles.

Plastic. A property of a soil which allows the soil to be deformed or molded

without cracking, or appreciable volume change.

Saturated soil. A soil in which the voids are filled with water. Saturation does

not require flow. Saturation, or near saturation, is necessary for the proper

use of instruments such as a pocket penetrometer or sheer vane.

Soil classification system. A method of categorizing soil and rock deposits in a

hierarchy of Stable Rock, Type A, Type B, and Type C, in decreasing order of

stability. The categories are determined based on an analysis of the properties

and performance characteristics of the deposits and the environmental conditions

of exposure.

Stable rock. Natural solid mineral matter that can be excavated with vertical

sides and remain intact while exposed.

Submerged soil. Soil which is underwater or is free seeping.

Type A soil. Cohesive soils with an unconfined, compressive strength of 1.5 ton

per square foot (tsf) or greater. Examples of cohesive soils are: clay, silty

clay, sandy clay, clay loam and, in some cases, silty clay loam and sandy clay

loam. Cemented soils such as caliche and hardpan are also considered Type A.

However, no soil is Type A if:

(1) The soil is fissured; or

(2) The soil is subject to vibration from heavy traffic, pile driving, or

similar effects; or

(3) The soil has been previously disturbed; or

(4) The soil is part of a sloped, layered system where the layers dip into the

excavation on a slope of four horizontal to one vertical (4H:1V) or greater; or

(5) The material is subject to other factors that would require it to be

classified as a less stable material.

Type B soil:

(1) Cohesive soil with an unconfined compressive strength greater than 0.5 tsf

but less than 1.5 tsf; or

(2) Granular cohesionless soils including: angular gravel (similar to crushed

rock), silt, silt loam, sandy loam and, in some cases, silty clay loam and sandy

clay loam.

(3) Previously disturbed soils except those which would otherwise be classed as

Type C soil.

(4) Soil that meets the unconfined compressive strength or cementation

requirements for Type A, but is fissured or subject to vibration; or

(5) Dry rock that is not stable; or

(6) Material that is part of a sloped, layered system where the layers dip into

the excavation on a slope less steep than four horizontal to one vertical

(4H:1V), but only if the material would otherwise be classified as Type B.

Type C soil:

(1) Cohesive soil with an unconfined compressive strength of 0.5 tsf or less; or

(2) Granular soils including gravel, sand, and loamy sand; or

(3) Submerged soil or soil from which water is freely seeping; or

(4) Submerged rock that is not stable, or

(5) Material in a sloped, layered system where the layers dip into the

excavation or a slope of four horizontal to one vertical (4H:1V) or steeper.

Unconfined compressive strength. The load per unit area at which a soil will

fail in compression. It can be determined by laboratory testing, or estimated in

the field using a pocket penetrometer, by thumb penetration tests, and other

methods.

Wet soil. Soil that contains significantly more moisture than moist soil, but in

such a range of values that cohesive material will slump or begin to flow when

vibrated. Granular material that would exhibit cohesive properties when moist

will lose those cohesive properties when wet.

(c) Requirements.

(1) Classification of soil and rock deposits. Each soil and rock deposit shall

be classified by a competent person as Stable Rock, Type A, Type B, or Type C in

accordance with the definitions set forth in paragraph (b) of this appendix.

(2) Basis of classification. The classification of the deposits shall be made

based on the results of at least one visual and at least one manual analysis.

Such analyses shall be conducted by a competent person using tests described in

paragraph (d) below, or in other approved methods of soil classification and

testing such as those adopted by the American Society for Testing Materials, or

the U.S. Department of Agriculture textural classification system.

(3) Visual and manual analyses. The visual and manual analyses, such as those

noted as being acceptable in paragraph (d) of this appendix, shall be designed

and conducted to provide sufficient quantitative and qualitative information as

may be necessary to identify properly the properties, factors, and conditions

affecting the classification of the deposits.

(4) Layered systems. In a layered system, the system shall be classified in

accordance with its weakest layer. However, each layer may be classified

individually where a more stable layer lies under a less stable layer.

(5) Reclassification. If, after classifying a deposit, the properties, factors,

or conditions affecting its classification change in any way, the changes shall

be evaluated by a competent person. The deposit shall be reclassified as

necessary to reflect the changed circumstances.

(d) Acceptable visual and manual tests.

(1) Visual tests. Visual analysis is conducted to determine qualitative

information regarding the excavation site in general, the soil adjacent to the

excavation, the soil forming the sides of the open excavation, and the soil

taken as samples from excavated material.

(A) Observe samples of soil that are excavated and soil in the sides of the

excavation. Estimate the range of particle sizes and the relative amounts of the

particle sizes. Soil that is primarily composed of fine-grained material is

cohesive material. Soil composed primarily of coarse-grained sand or gravel is

granular material.

(B) Observe soil as it is excavated. Soil that remains in clumps when excavated

is cohesive. Soil that breads up easily and does not stay in clumps is granular.

(C) Observe the side of the opened excavation and the surface area adjacent to

the excavation. Crack-like openings such as tension cracks could indicate

fissured material. If chunks of soil spall off a vertical side, the soil could

be fissured. Small spalls are evidence of moving ground and are indications of

potentially hazardous situations.

(D) Observe the area adjacent to the excavation and the excavation itself for

evidence of existing utility and other underground structures, and to identify

previously disturbed soil.

(E) Observe the opened side of the excavation to identify layered systems.

Examine layered systems to identify if the layers slope toward the excavation.

Estimate the degree of slope of the layers.

(F) Observe the area adjacent to the excavation and the sides of the opened

excavation for evidence of surface water, water seeping from the sides of the

excavation, or the location of the level of the water table.

(G) Observe the area adjacent to the excavation and the area within the

excavation for sources of vibration that may affect the stability of the

excavation face.

(2) Manual tests. Manual analysis of soil samples in conducted to determine

quantitative as well as qualitative properties of soil and to provide more

information in order to classify soil properly.

(A) Plasticity. Mold a moist or wet sample of soil into a ball and attempt to

roll it into threads as thin as 1/8-inch in diameter. Cohesive material can be

successfully rolled into threads without crumbling. For example, if at least a

two inch length of 1/8-inch thread can be held on one end without tearing, the

soil is cohesive.

(B) Dry strength. If the soil is dry and crumbles on its own or with moderate

pressure into individual grains or fine powder, it is granular (any combination

of gravel, sand, or silt). If the soil is dry and falls into clumps which break

up into smaller clumps, but the smaller clumps can only be broken up with

difficulty, it may be clay in any combination with gravel, sand or silt. If the

dry soil breads into clumps which do not break up into small clumps and which

can only be broken with difficulty, and there is no visual indication the soil

is fissured, the soil may be considered un-fissured.

(C) Thumb penetration. The thumb penetration test can be used to estimate the

unconfined compressive strength of cohesive soils. Type A soils with an

unconfined compressive strength of 1.5 tsf can be readily indented by the thumb;

however, they can be penetrated by the thumb only with very great effort. Type C

soils with an unconfined compressive strength of 0.5 tsf can be easily

penetrated several inches by the thumb, and can be molded by light finger

pressure. This test should be conducted on an undisturbed soil sample, such as a

large clump of spoil, as soon as practicable after excavation to keep to a

minimum the effects of exposure to drying influences (rain, flooding), the

classification of the soil must be changed accordingly.

(D) Other strength tests. Estimates of unconfined compressive strength of soils

can also be obtained by use of a pocket penetrometer or by using a hand-operated

shear vane.

(E) Drying test. The basic purpose of the drying test is to differentiate

between cohesive material with fissures, un-fissured cohesive material, and

granular material. The procedure for the drying test involves drying a sample of

soil that is approximately one inch thick and six inches in diameter until it is

thoroughly dry:

1. If the sample develops cracks as it dries, significant fissures are

indicated.

2. Samples that dry without cracking are to be broken by hand. If considerable

force is necessary to bread a sample, the soil has significant cohesive material

content. The soil can be classified as an un-fissured cohesive material and the

unconfined compressive strength should be determined.

3. If a sample breaks easily by hand, it is either a fissured cohesive material

or a granular material. To distinguish between the two, pulverize the dried

clumps of the sample by hand or by stepping on them. If the clumps do not

pulverize easily, the material is cohesive with fissures. If they pulverize

easily into very small fragments, the material is granular.

### Requirements for Protective Systems, Appendix B

Sloping and Benching

(a) Scope and application. This appendix contains specifications for sloping and

benching when used as methods of protecting employees working in excavations

from cave-ins. The requirements of this appendix apply when the design of

sloping and benching protective systems is to be performed in accordance with

the requirements set forth in Section 1541.1(b).

(b) Definitions.

Actual slope means the slope to which an excavation face is excavated.

Distress means that the soil is in a condition where a cave-in is imminent or is

likely to occur. Distress is evidenced by such phenomena as the development of

fissures in the face of or adjacent to an open excavation; the subsidence of the

edge of an excavation; the slumping of material from the face or the bulging or

heaving of material from the bottom of an excavation; the spalling of material

from the face of an excavation; and ravelling, i.e., small amounts of material

such as pebbles or little clumps of material suddenly separating from the face

of an excavation and trickling or rolling down into the excavation.

Maximum allowable slope means the steepest incline of an excavation face that is

acceptable for the most favorable site conditions as protection against

cave-ins, and is expressed as the ratio of horizontal distance to vertical rise

(H:V),

Short term exposure means a period of time less than or equal to 24 hours that

an excavation is open.

(c) Requirements.

(1) Soil classification. Soil and rock deposits shall be classified in

accordance with Appendix A to Section 1541.1.

(2) Maximum allowable slope. The maximum allowable slope for a soil or rock

deposit shall be determined from Table B-1 of this appendix.

(3) Actual slope.

(A) The actual slope shall not be steeper than the maximum allowable slope.

(B) The actual slope shall be less steep than the maximum allowable slope, when

there are signs of distress. If that situation occurs, the slope shall be cut

back to an actual slope which is at least 1/2 horizontal to one vertical

(1/2H:1V) less steep than the maximum allowable slope.

(C) When surcharge loads from stored material or equipment, operating equipment,

or traffic are present, a competent person shall determine the degree to which

the actual slope must be reduced below the maximum allowable slope, and shall

assure that such reduction is achieved. Surcharge loads from adjacent structures

shall be evaluated in accordance with Section 1541(i).

(4) Configurations. Configurations of sloping and benching systems shall be in

accordance with Figure B-1.

TABLE B-1

MAXIMUM ALLOWABLE SLOPES

______________________________________________________________

SOIL OR ROCK TYPE  | MAXIMUM ALLOWABLE SLOPES (H:V)(1) FOR

| EXCAVATIONS LESS THAN 20 FEET DEEP(3)

____________________|___________________________________

STABLE ROCK         | VERTICAL (90 Deg.)

TYPE A (2)          | 3/4:1    (53 Deg.)

TYPE B              | 1:1      (45 Deg.)

TYPE C              | 1 1/2:1  (34 Deg.)

____________________|____________________________________

NOTES:

1. Numbers shown in parentheses next to maximum allowable slopes are angles expressed in degrees from the horizontal. Angles have been rounded off.

2. A short-term maximum allowable slope of 1/2H:1V (63 degrees) is allowed in excavations in Type A soil that are 12 feed (3.67 m) or less in depth. Short-term maximum allowable slopes for excavations greater than 12 feet (3.67 m) in depth shall be 3/4H:1V (53 degrees).

3. Sloping or benching for excavations greater than 20 feet deep shall be designed by a registered professional engineer.

Figure B-1

Slope Configurations

(All slopes stated below are in the horizontal to vertical ratio)

B-1.1 Excavations made in Type A soil

1. All simple slope excavations 20 feet or less in depth shall gave a maximum

allowable slope of 3/4:1.

Simple Slope-General

Exception: Simple slope excavations which are open 24 hours or less (short term)

and which are 12 feet or less in depth shall have a maximum allowable slope of

1/2:1.

Simple Slope- Short Term

2.All benched excavations 20 feet or less in depth shall have a maximum

allowable slope of 3/4 to 1 and maximum bench dimensions as follows:

Simple Bench

Multiple Bench

3. All excavations 8 feet or less in depth which have unsupported vertically

sided lower portions shall have a maximum vertical side 3 1/2 feet.

Unsupported Vertically Sided Lower Portion-Maximum 8 Feet Depth

All excavations more than 8 feet but not more than 12 feet in depth with

unsupported vertically sided lower portions shall have a maximum allowable slope

of 1:1 and a maximum vertical side of 3 1/2 feet.

Unsupported Vertically Sided Lower Portion-Maximum 12 Feet in Depth

All excavations 20 feet or less in depth which have vertically sided lower

portions that are supported or shielded shall have a maximum allowable slope of

3/4:1. The support or shield system must extend at least 18 inches above the top

of the vertical side

Supported or Shielded Vertically Sided Lower Portion

B- 1.2 Excavations Made in Type B Soil

1. All simple slope excavations 20 feet or less in depth shall have a maximum

allowable slope of 1:1.

Simple Slope

2. All benched excavations 20 feet or less in depth shall have a maximum

allowable slope of 1:1 and maximum bench dimensions as follows:

Single Bench

Multiple Bench

3. All excavations 20 feet or less in depth which have vertically sided lower

portions shall be shielded or supported to a height at least 18 inches above the

top of the vertical side. All such excavations shall have a maximum allowable

slope of 1:1

Vertically Sided Lower Portion

B- 1.3 Excavations Made in Type C Soil

1. All simple slope excavations 20 feet or less in depth shall have a maximum

allowable slope of 1 1/2:1

Simple Slope

2.All excavations 20 feet or less in depth which have vertically sided lower

portions shall be shielded or supported to a height at least 18 inches above the

top of the vertical side. All such excavations shall have a maximum allowable

slope of 1 1/2:1.

Vertically Sided Lower Portion

3. All other sloped excavations shall be in accordance with the other options

permitted in 1541.1(b)

B- 1.4 Excavations Made in Layered Soil

1. All excavations 20 feet or less in depth made in layered soils shall have a

maximum allowable slope for each layer as set forth below:

B OVER A

C OVER A

C OVER B

A OVER B

A OVER C

B OVER C

2. All other sloped excavations shall be in accordance with the other options

permitted in (b).

### Requirements for Protective Systems, Appendix C

Timber Shoring for Trenches

(a) Scope. This appendix contains information that can be used when timber

shoring is provided as a method of protection from cave-ins in trenches that do

not exceed 20 feet in depth. This appendix must be used when design of timber

shoring protective systems is to be performed in accordance with Section

(c)(1). Other timber shoring configurations; other systems of support such

as hydraulic and pneumatic systems; and other protective systems such as

sloping, benching, shielding, and freezing systems must be designed in

accordance with the requirements set forth in Section (b) and (c).

(b) Soil Classification. In order to use the data presented in this appendix,

the soil type or types in which the excavation is made must first be determined

using the soil classification method set forth in Article 6.

(c) Presentation of Information.

Information is presented in several forms as follows:

(1) Information is presented in tabular form in Tables C-1.1, C-1.2 and C-1.3,

and Tables C-2.1, C-2.2 and C-2.3 following Section (g) of Appendix C. Each

table presents the minimum sizes of timber members to use in a shoring system,

and each table contains data only for the particular soil type in which the

excavation or portion of the excavation is made. The data are arranged to allow

the user the flexibility to select from among several acceptable configurations

of members based on varying the horizontal spacing of the crossbraces. Stable

rock is exempt from shoring requirements and therefore, no data are presented

for this condition.

(2) Information concerning the basis of the tabular data and the limitations of

the data is presented in Section (d) of this appendix, and on the tables

themselves.

(3) Information explaining the use of the tabular data is presented in Section (e) of this appendix.

(4) Information illustrating the use of the tabular data is presented in Section

(f) of this appendix.

(5) Miscellaneous notations regarding Tables C-1.1 through C-1.3 and Tables

C-2.1 through C-2.3 are presented in Section (g) of this appendix.

(d) Basis and limitations of the data.

(1) Dimensions of timber members.

(A) The sizes of the timber members listed in Tables C-1.1 through C-1.3 are

taken from the National Bureau of Standards (NBS) report, ``Recommended

Technical Provisions for Construction Practice in Shoring and Sloping of

Trenches and Excavations.'' In addition, where NBS did not recommend specific

sizes of members, member sizes are based on an analysis of the sizes required

for use by existing codes and on empirical practice.

(B) The required dimensions of the members listed in Tables C-1.1 through C-1.3

refer to actual dimensions and not nominal dimensions of the timber. Companies

wanting to use nominal size shoring are directed to Tables C-2.1 through C-2.3,

or have this choice under Section 1541.1(c)(3).

(2) Limitation of application.

(A) It is not intended that the timber shoring specification apply to every

situation that may be experienced in the field. These data were developed to

apply to the situations that are most commonly experienced in current trenching

practice. Shoring systems for use in situations that are not covered by the data

in this appendix must be designed as specified in Section 1541.1(c).

(B) When any of the following conditions are present, the members specified in

the tables are not considered adequate. Either an alternate timber shoring

system must be designed or another type of protective system designed in

accordance with Section 1541.1.

1. When loads imposed by structures or by stored material adjacent to the trench

weigh in excess of the load imposed by a two-foot soil surcharge. The term

``adjacent'' as used here means the area within a horizontal distance from the

edge of the trench equal to the depth of the trench.

2. When vertical loads imposed on crossbraces exceed a 240-pound gravity load

distributed on a one-foot section of the center of the crossbrace.

3. When surcharge loads are present from equipment weighing in excess of 20,000 pounds.

4. When only the lower portion of a trench is shored and the remaining portion

of the trench is sloped or benched unless: The sloped portion is sloped at an

angle less steep than three horizontal to one vertical; or the members are

selected from the tables for use at a depth which is determined from the top of

the overall trench, and not from the toe of the sloped portion.

(e) Use of Tables. The members of the shoring system that are to be selected

using this information are the crossbraces, the uprights, and the wales, where

wales are required. Minimum sizes of members are specified for use in different

types of soil. There are six tables of information, two for each soil type. The

soil type must first be determined in accordance with the soil classification

system described in Appendix A. Using the appropriate table, the selection of

the size and spacing of the members is then made. The selection is based on the

depth and width of the trench where the members are to be installed and, in most

instances, the selection is also based on the horizontal spacing of the

crossbraces. Instances where a choice of horizontal spacing of crossbracing is

available, the horizontal spacing of the crossbraces must be chosen by the user

before the size of any member can be determined. When the soil type, the width

and depth of the trench, and the horizontal spacing of the crossbraces, the size

and vertical spacing of the crossbraces are known, the size and vertical spacing

of the crossbraces, the size and vertical spacing of the wales, and the size and

horizontal spacing of the uprights can be read from the appropriate table.

(f) Examples to Illustrate the Use of Tables C-1.1 through C-1.3.

(1) Example 1.

A trench dug in Type A soil is 13 feet deep and five feet wide. From Table

C-1.1, four acceptable arrangements of timber can be used.

Arrangement #1

Space 4X4 crossbraces at six feet horizontally and four feet vertically. Wales

are not required.

Space 3X8 uprights at six feet horizontally. This arrangement is commonly called

``skip shoring.''

Arrangement #2

Space 4X6 crossbraces at eight feet horizontally and four feet vertically.

Space 8X8 wales at four feet vertically.

Space 2X6 uprights at four feet horizontally.

Arrangement #3

Space 6X6 crossbraces at 10 feet horizontally and four feet vertically.

Space 8X10 wales at four feet vertically.

Space 2X6 uprights at five feet horizontally.

Arrangement #4

Space 6X6 crossbraces at 12 feet horizontally and four feet vertically.

Space 10X10 wales at four feet vertically.

Space 3X8 uprights at six feet horizontally.

(2) Example 2.

A trench dug in Type B soil is 13 feet deep and five feet wide. From Table C-1.2

three acceptable arrangements of members are listed.

Arrangement #1

Space 6X6 crossbraces at six feet horizontally and five feet vertically.

Space 8X8 wales at five feet vertically.

Space 2X6 uprights at two feet horizontally.

Arrangement #2

Space 6X8 crossbraces at eight feet horizontally and five feet vertically.

Space 10X10 wales at five feet vertically.

Space 2X6 uprights at two feet horizontally.

Arrangement #3

Space 8X8 crossbraces at 10 feet horizontally and five feet vertically.

Space 10X12 wales at five feet vertically.

Space 2X6 uprights at two feet vertically.

(3) Example 3.

A trench dug in Type C soil is 13 feet deep and five feet wide.

From Table C-1.3 two acceptable arrangements of members can be used.

Arrangement #1

Space 8X8 crossbraces at six feet horizontally and five feet vertically.

Space 10X12 wales at five feet vertically.

Position 2X6 uprights as closely together as possible.

If water must be retained use special tongue and groove uprights to form tight

sheeting.

Arrangement #2

Space 8X10 crossbraces at eight feet horizontally and five feet vertically.

Space 12X12 wales at five feet vertically.

Position 2X6 uprights in a close sheeting configuration unless water pressure

must be resisted. Tight sheeting must be used where water must be retained.

(4) Example 4.

A trench dug in Type C soil is 20 feet deep and 11 feet wide. The size and

spacing of members for the section of trench that is over 15 feet in depth is

determined using Table C-1.3. Only one arrangement of members is provided.

Space 8X10 crossbraces at six feet horizontally and five feet vertically.

Space 12X12 wales at five feet vertically.

Use 3X6 tightsheeting.

Use of Tables C-2.1 through C-2.3 would follow the same procedures.

(g) Notes for all Tables.

1. Members sizes at spacings other than indicated are to be determined as

specified in Section 1541.1(c), ``Design of Protective Systems.''

2. When conditions are saturated or submerged use Tight Sheeting. Tight Sheeting refers to the use of specially-edged timber planks (e.g., tongue and groove) at least three inches thick, steel sheet piling, or similar construction that when driven or placed in position provide a tight wall to resist the lateral pressure

of water and to prevent the loss of backfill material. Close Sheeting refers to

the placement of planks side-by-side allowing as little space as possible

between them.

3. All spacing indicated is measured center to center.

4.. Wales to be installed with greater dimension horizontal.

5. If the vertical distance from the center of the lowest crossbrace to the

bottom of the trench exceeds two and one-half feet, uprights shall be firmly

embedded or a mudsill shall be used. Where uprights are embedded, the vertical

distance from the center of the lowest crossbrace to the bottom of the trench

shall not exceed 36 inches. When mudsills are used, the vertical distance shall

not exceed 42 inches. Mudsills are wales that are installed at the toe of the

trench side.

6. Trench jacks may be used in lieu of or in combination with timber

crossbraces.

7. Placement of crossbraces. When the vertical spacing of crossbraces is four

feet, place the top crossbrace no more than two feet below the top of the

trench. When the vertical spacing of crossbraces is five feet, place the top

crossbrace no more than 2.5 feet below the top of the trench.

TABLE C-1.1

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE A  P(a) = 25 X H + 72 psf (2 ft Surcharge)

____________________________________________________________________

|

|        SIZE (ACTUAL) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |

|                    CROSS BRACES

OF  |_____________________________________________________________

|       |                                       |

| HORIZ.|         WIDTH OF TRENCH (FEET)        |   VERT.

TRENCH|SPACING|_______________________________________|  SPACING

|       |       |       |       |       |       |

|       | UP TO | UP TO | UP TO | UP TO | UP TO |

(FEET)| (FEET)|   4   |   6   |   9   |   12  |   15  |  (FEET)

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  4X4  | 4X4   | 4X6   |  6X6  |  6X6  |   4

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   | UP TO |       |       |       |       |       |

|   8   |  4X4  | 4X4   | 4X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  4X6  | 4X6   | 4X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   12  |  4X6  | 4X6   | 6X6   |  6X6  |  6X6  |   4

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  4X4  | 4X4   | 4X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  4X6  | 4X6   | 6X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   4

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   12  |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   4

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   4

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   4

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  8X8  | 8X8   | 8X8   |  8X8  |  8X10 |   4

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   12  |  8X8  | 8X8   | 8X8   |  8X8  |  8X10 |   4

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

TABLE C-1.1

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE A  P(a) = 25 X H + 72 psf (2 ft Surcharge)

[Continued]

____________________________________________________________________

|

|        SIZE (ACTUAL) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |               |

|   WALES       |             UPRIGHTS

OF  |_______________|_____________________________________________

|       |       |

TRENCH|       | VERT. |    MAXIMUM ALLOWABLE HORIZONTAL SPACING

|       |SPACING|               (FEET)

| SIZE  |       |_____________________________________________

|       |       |       |       |       |       |

(FEET)| (IN)  |(FEET) | CLOSE |   4   |   5   |   6   |   8

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|  Not  |       |       |       |       |       |

| Req'd |  ---  |       |       |       |  2X6  |

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   |  Not  |       |       |       |       |       |

| Req'd |  ---  |       |       |       |       |   2X8

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   |       |       |  2X6  |       |

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   |       |       |       |  2X6  |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|  Not  |       |       |       |       |       |

| Req'd | ---   |       |       |       |  3X8  |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   |       | 2X6   |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   4   |       |       |  2X6  |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X10 |   4   |       |       |       |  3X8  |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  6X8  |   4   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   4   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X10 |   4   | 3X6   |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

* Mixed oak or equivalent with a bending strength not less than 850 psi.

** Manufactured members of equivalent strength may be substituted for wood.

TABLE C-1.2

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE B  P(a) = 45 X H + 72 psf (2 ft Surcharge)

____________________________________________________________________

|

|        SIZE (ACTUAL) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |

|                    CROSS BRACES

OF  |_____________________________________________________________

|       |                                       |

| HORIZ.|         WIDTH OF TRENCH (FEET)        |   VERT.

TRENCH|SPACING|_______________________________________|  SPACING

|       |       |       |       |       |       |

|       | UP TO | UP TO | UP TO | UP TO | UP TO |

(FEET)| (FEET)|   4   |   6   |   9   |   12  |   15  |  (FEET)

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  4X6  | 4X6   | 6X6   |  6X6  |  6X6  |   5

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   | UP TO |       |       |       |       |       |

|   8   |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  6X8  | 6X8   | 6X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  8X8  | 8X8   | 8X8   |  8X8  |  8X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X8  | 6X8   | 6X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  8X8  | 8X8   | 8X8   |  8X8  |  8X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  8X10 | 8X10  | 8X10  |  8X10 | 10X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

TABLE C-1.2

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE B  P(a) = 45 X H + 72 psf (2 ft Surcharge)

[Continued]

____________________________________________________________________

|

|        SIZE (ACTUAL) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |               |

|   WALES       |             UPRIGHTS

OF  |_______________|_____________________________________________

|       |       |

TRENCH|       | VERT. |    MAXIMUM ALLOWABLE HORIZONTAL SPACING

|       |SPACING|               (FEET)

| SIZE  |       |_____________________________________________

|       |       |       |       |       |       |

(FEET)| (IN)  |(FEET) | CLOSE |   2   |   3   |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  6X8  |   5   |       |       |  2X6  |       |

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   |       |       |       |       |       |       |

|  8X10 |   5   |       |       |  2X6  |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X10 |   5   |       |       |  2X6  |       |

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   5   |       |  2X6  |       |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X10 |   5   |       |  2X6  |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   |       |  2X6  |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   5   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 12X12 |   5   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

* Mixed oak or equivalent with a bending strength not less than 850 psi.

** Manufactured members of equivalent strength may be substituted for wood.

TABLE C-1.3

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE C  P(a) = 80 X H + 72 psf (2 ft Surcharge)

____________________________________________________________________

|

|        SIZE (ACTUAL) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |

|                    CROSS BRACES

OF  |_____________________________________________________________

|       |                                       |

| HORIZ.|         WIDTH OF TRENCH (FEET)        |   VERT.

TRENCH|SPACING|_______________________________________|  SPACING

|       |       |       |       |       |       |

|       | UP TO | UP TO | UP TO | UP TO | UP TO |

(FEET)| (FEET)|   4   |   6   |   9   |   12  |   15  |  (FEET)

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X8  | 6X8   | 6X8   |  8X8  |  8X8  |   5

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   | UP TO |       |       |       |       |       |

|   8   |  8X8  | 8X8   | 8X8   |  8X8  |  8X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  8X10 | 8X10  | 8X10  |  8X10 | 10X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  8X8  | 8X8   | 8X8   |  8X8  |  8X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  8X10 | 8X10  | 8X10  |  8X10 | 10X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  8X10 | 8X10  | 8X10  |  8X10 | 10X10 |   5

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

TABLE C-1.3

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE C  P(a) = 80 X H + 72 psf (2 ft Surcharge)

[Continued]

____________________________________________________________________

|

|        SIZE (ACTUAL) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |               |

|   WALES       |             UPRIGHTS

OF  |_______________|_____________________________________________

|       |       |

TRENCH|       | VERT. |    MAXIMUM ALLOWABLE HORIZONTAL SPACING

|       |SPACING|               (FEET)

| SIZE  |       |_____________________________________________

|       |       |       |       |       |       |

(FEET)| (IN)  |(FEET) | CLOSE |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   5   | 2X6   |       |       |       |

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   |       |       |       |       |       |       |

| 10X12 |   5   | 2X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 12X12 |   5   | 2X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   | 2X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 12X12 |   5   | 2X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

| 12X12 |   5   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

* Mixed oak or equivalent with a bending strength not less than 850 psi.

** Manufactured members of equivalent strength may be substituted for wood.

TABLE C-2.1

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE A  P(a) = 25 X H + 72 psf (2 ft Surcharge)

____________________________________________________________________

|

|        SIZE (S4S) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |

|                    CROSS BRACES

OF  |_____________________________________________________________

|       |                                       |

| HORIZ.|         WIDTH OF TRENCH (FEET)        |   VERT.

TRENCH|SPACING|_______________________________________|  SPACING

|       |       |       |       |       |       |

|       | UP TO | UP TO | UP TO | UP TO | UP TO |

(FEET)| (FEET)|   4   |   6   |   9   |   12  |   15  |  (FEET)

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  4X4  | 4X4   | 4X4   |  4X4  |  4X6  |   4

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   | UP TO |       |       |       |       |       |

|   8   |  4X4  | 4X4   | 4X4   |  4X6  |  4X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  4X6  | 4X6   | 4X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   12  |  4X6  | 4X6   | 4X6   |  6X6  |  6X6  |   4

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  4X4  | 4X4   | 4X4   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  4X6  | 4X6   | 4X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  6X6  | 6X6   | 6X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   12  |  6X6  | 6X6   | 6X6   |  6X6  |  6X6  |   4

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X6  | 6X6   | 6X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  6X6  | 6X6   | 6X6   |  6X6  |  6X6  |   4

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  6X6  | 6X6   | 6X6   |  6X6  |  6X8  |   4

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   12  |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   4

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

TABLE C-2.1

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE A  P(a) = 25 X H + 72 psf (2 ft Surcharge)

[Continued]

____________________________________________________________________

|

|        SIZE (S4S) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |               |

|   WALES       |             UPRIGHTS

OF  |_______________|_____________________________________________

|       |       |

TRENCH|       | VERT. |    MAXIMUM ALLOWABLE HORIZONTAL SPACING

|       |SPACING|               (FEET)

| SIZE  |       |_____________________________________________

|       |       |       |       |       |       |

(FEET)| (IN)  |(FEET) | CLOSE |   4   |   5   |   6   |   8

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|  Not  | Not   |       |       |       |       |

| Req'd | Req'd |       |       |       |  4X6  |

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   |  Not  |  Not  |       |       |       |       |

| Req'd | Req'd |       |       |       |       |   4X8

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   |       |       |  4X6  |       |

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   |       |       |       |  4X6  |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|  Not  |  Not  |       |       |       |       |

| Req'd | Req'd |       |       |       |  4X10 |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  6X8  |   4   |       | 4X6   |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   |       |       |  4X8  |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   4   |       | 4X6   |       | 4X10  |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  6X8  |   4   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   4   | 3X6   | 4x12  |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   4   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X12 |   4   | 3X6   | 4x12  |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

* Douglas fir or equivalent with a bending strength not less than 1500 psi.

** Manufactured members of equivalent strength may be substituted for wood.

TABLE C-2.2

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE B  P(a) = 45 X H + 72 psf (2 ft Surcharge)

____________________________________________________________________

|

|        SIZE (S4S) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |

|                    CROSS BRACES

OF  |_____________________________________________________________

|       |                                       |

| HORIZ.|         WIDTH OF TRENCH (FEET)        |   VERT.

TRENCH|SPACING|_______________________________________|  SPACING

|       |       |       |       |       |       |

|       | UP TO | UP TO | UP TO | UP TO | UP TO |

(FEET)| (FEET)|   4   |   6   |   9   |   12  |   15  |  (FEET)

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  4X6  | 4X6   | 4X6   |  6X6  |  6X6  |   5

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   | UP TO |       |       |       |       |       |

|   8   |  4X6  | 4X6   | 6X6   |  6X6  |  6X6  |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  4X6  | 4X6   | 6X6   |  6X6  |  6X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X6  | 6X6   | 6X6   |  6X8  |  6X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  6X8  | 6X8   | 6X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  6X8  | 6X8   | 8X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X8  | 6X8   | 6X8   |  6X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   |  6X8  | 6X8   | 6X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  8X8  | 8X8   | 8X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

TABLE C-2.2

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE B  P(a) = 45 X H + 72 psf (2 ft Surcharge)

[Continued]

____________________________________________________________________

|

|        SIZE (S4S) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |               |

|   WALES       |             UPRIGHTS

OF  |_______________|_____________________________________________

|       |       |

TRENCH|       | VERT. |    MAXIMUM ALLOWABLE HORIZONTAL SPACING

|       |SPACING|               (FEET)

| SIZE  |       |_____________________________________________

|       |       |       |       |       |       |

(FEET)| (IN)  |(FEET) | CLOSE |   2   |   3   |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |  3X12 |       |

|  6X8  |   5   |       |       |  4X8  |       | 4X12

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   |       |       |       |       |       |       |

|  8X8  |   5   |       |  3X8  |       |  4X8  |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   5   |       |       |  4X8  |       |

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X8  |   5   |  3X6  |  4X10 |       |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X10 |   5   |  3X6  |  4X10 |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   |  3X6  |  4X10 |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|  8X10 |   5   | 4X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   | 4X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 12X12 |   5   | 4X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

* Douglas fir or equivalent with a bending strength not less than 1500 psi.

** Manufactured members of equivalent strength may be substituted for wood.

TABLE C-2.3

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE C  P(a) = 80 X H + 72 psf (2 ft Surcharge)

____________________________________________________________________

|

|        SIZE (S4S) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |

|                    CROSS BRACES

OF  |_____________________________________________________________

|       |                                       |

| HORIZ.|         WIDTH OF TRENCH (FEET)        |   VERT.

TRENCH|SPACING|_______________________________________|  SPACING

|       |       |       |       |       |       |

|       | UP TO | UP TO | UP TO | UP TO | UP TO |

(FEET)| (FEET)|   4   |   6   |   9   |   12  |   15  |  (FEET)

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   |  6X6  | 6X6   | 6X6   |  6X6  |  8X8  |   5

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   | UP TO |       |       |       |       |       |

|   8   |  6X6  | 6X6   | 6X6   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   10  |  6X6  | 6X6   | 8X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   | 6X8   | 6X8   | 6X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   8   | 8X8   | 8X8   | 8X8   |  8X8  |  8X8  |   5

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

| UP TO |       |       |       |       |       |

|   6   | 8X8   | 8X8   | 8X8   | 8X10  | 8X10  |   5

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

| See   |       |       |       |       |       |

| Note 1|       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

TABLE C-2.3

TIMBER TRENCH SHORING -- MINIMUM TIMBER REQUIREMENTS *

SOIL TYPE C  P(a) = 80 X H + 72 psf (2 ft Surcharge)

[Continued]

____________________________________________________________________

|

|        SIZE (S4S) AND SPACING OF MEMBERS **

|_____________________________________________________________

DEPTH |               |

|   WALES       |             UPRIGHTS

OF  |_______________|_____________________________________________

|       |       |

TRENCH|       | VERT. |    MAXIMUM ALLOWABLE HORIZONTAL SPACING

|       |SPACING|               (FEET)

| SIZE  |       |_____________________________________________

|       |       |       |       |       |       |

(FEET)| (IN)  |(FEET) | CLOSE |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

| 8X8   |   5   | 3X6   |       |       |       |

5   |_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

TO   |       |       |       |       |       |       |

| 10X10 |   5   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   | 3X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X10 |   5   | 4X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

10   |       |       |       |       |       |       |

|       |       |       |       |       |       |

| 12X12 |   5   | 4X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|       |       |       |       |       |       |

|       |       |       |       |       |       |

| 10X12 |   5   | 4X6   |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

15   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

TO   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

|_______|_______|_______|_______|_______|_______|_____________

20   |       |       |       |       |       |       |

|       |       |       |       |       |       |

|       |       |       |       |       |       |

______|_______|_______|_______|_______|_______|_______|_____________

|

OVER |   SEE NOTE 1

20  |

______|_____________________________________________________________

* Douglas fir or equivalent with a bending strength not less than 1500 psi.

** Manufactured members of equivalent strength may be substituted for wood.

### Requirements for Protective Systems, Appendix D

Aluminum Hydraulic Shoring for Trenches

(a) Scope. This appendix contains information that can be used when aluminum

hydraulic shoring is provided as a method of protection against cave-ins in

trenches that do not exceed 20 feet in depth. This appendix must be used when

design of the aluminum hydraulic protective system cannot be performed in

accordance with Section 1541.1(c)(2).

(b) Soil Classification. In order to use data presented in this appendix, the

soil type or types in which the excavation is made must first be determined

using the soil classification method set forth in Appendix A of this Article.

(c) Presentation of Information. Information is presented in several forms as

follows:

(1) Information is presented in tabular form in Tables D-1.1, D-1.2, D-1.3 and

D-1.4. Each table presents the maximum vertical and horizontal spacings that may

be used with various aluminum member sizes and various hydraulic cylinder sizes.

Each table contains data only for the particular soil type in which the

excavation or portion of the excavation is made. Tables D-1.1 and D-1.2 are for

vertical shores in Types A and B soil. Tables D-1.3 and D-1.4 are for horizontal

waler systems in Types B and C soil.

(2) Information concerning the basis of the tabular data and the limitations of

the data is presented in Section (d) of this appendix.

(3) Information explaining the use of the tabular data is presented in Section

(e) of this appendix.

(4) Information illustrating the use of the tabular data is presented in Section

(f) of this appendix.

(5) Miscellaneous notations (footnotes) regarding Table D-1.1 through D-1.4 are

presented in Section (g) of this appendix.

(6) Figures, illustrating typical installations of hydraulic shoring, are

included just prior to the Tables. The illustrations page is entitled ``Aluminum

Hydraulic Shoring: Typical Installations.''

(d) Basis and limitations of the data.

(1) Vertical shore rails and horizontal wales are those that meet the Section

Modulus requirements in the D-1 Tables. Aluminum material is 6061-T6 or material

of equivalent strength and properties.

(2) Hydraulic cylinders specifications.

(A) 2-inch cylinders shall be a minimum 2-inch inside diameter with a minimum

safe working capacity of no less than 18,000 pounds axial compressive load at

maximum extension. Maximum extension is to include full range of cylinder

extensions as recommended by product manufacturer.

(B) 3-inch cylinders shall be minimum 3-inch inside diameter with a safe working

capacity of not less than 30,000 pounds axial compressive load at extensions as

recommended by product manufacturer.

(3) Limitation of application.

(A) It is not intended that the aluminum hydraulic specification apply to every

situation that may be experienced in the field. These data were developed to

apply to the situations that are most commonly experienced in current trenching

practice. Shoring systems for use in situations that are not covered by the data

in this appendix must be otherwise designed as specified in Section 1541.1(c).

(B) When any of the following conditions are present, the members specified in

the Tables are no considered adequate. In this case, an alternative aluminum

hydraulic shoring system or other type of protective system must be designed in

accordance with this section.

1. When vertical loads imposed on crossbraces exceed a 100 pound gravity load

distributed on a one foot section of the center of the hydraulic cylinder.

2. When surcharge loads are present from equipment weighing in excess of 20,000

pounds.

3. When only the lower portion of a trench is shored and the remaining portion

of the trench is sloped or benched unless: The sloped portion is sloped at an

angle less steep than three horizontal to one vertical; or the members are

selected from the tables for use at a depth which is determined from the top of

the overall trench, and not from the toe of the sloped portion.

(e) Use of Tables D-1.1, D-1.2, D-1.3 and D-1.4. The members of the shoring

system that are to be selected using this information are the hydraulic

cylinders, and either the vertical shores or the horizontal wales. When a waler

system is used the vertical timber sheeting to be used is also selected from

these tables. The Tables D-1.1 and D-1.2 for vertical shores are used in Type A

and B soils that do not require sheeting. Type B soils that may require

sheeting, and Type C soils that always require sheeting, are found in the

horizontal wale Tables D-1.3 and D-1.4. The soil type must first be determined

in accordance with the soil classification system described in Appendix A to

this Section. Using the appropriate table, the selection of the size and

spacing of the members is made. The selection is based on the depth and width of

the trench where the members are to be installed. In these tables the vertical

spacing is held constant at four feet on center. The tables show the maximum

horizontal spacing of cylinders allowed for each size of wale in the waler

system tables, and in the vertical shore tables, the hydraulic cylinder

horizontal spacing is the same as the vertical shore spacing.

(f) Example to Illustrate the Use of the Tables:

(1) Example 1.

A trench dug in Type A soil is 6 feet deep and 3 feet wide. From Table D-1.1:

Find vertical shores and 2 inch diameter cylinders spaced 8 feet on center

(o.c.) horizontally and 4 feet on center (o.c.) vertically. (See Figures 1 & 3

for typical installations.)

(2) Example 2:

A trench is dug in Type B soil that does not require sheeting, 13 feet deep and

5 feet wide. From Table D-1.2: Find vertical shores and 2 inch diameter

cylinders spaced 6.5 feet o.c. horizontally and 4 feet o.c. vertically. (See

Figures 1 & 3 for typical installations.)

(3) Example 3:

A trench is dug in Type B soil that does not require sheeting, but does

experience some minor raveling of the trench face. The trench is 16 feet deep

and 9 feet wide. From Table D-1.2: Find vertical shores and 2 inch diameter

cylinder (with special oversleeves as designated by footnote #2) spaced 5.5 feet

o.c. horizontally and 4 feet o.c. vertically. Plywood (per footnote (g)(7) to

the D-1 Table) should be used behind the shores. (See Figures 2 & 3 for typical

installations.)

(4) Example 4:

A trench is dug in previously disturbed Type B soil, with characteristics of a

Type C soil, and will require sheeting. The trench is 18 feet deep, and 12 feet

wide. 8 foot horizontal spacing between cylinders is desired for working space.

From Table D-1.3: Find horizontal wale with a section modulus of 14.0 spaced at

4 feet o.c. vertically and 3 inch diameter cylinder spaced at 9 feet maximum

o.c. horizontally, 3 x 12 timber sheeting is required at close spacing

vertically. (See Figure 4 for typical installation.)

(5) Example 5:

A trench is dug in Type C soil, 9 feet deep and 4 feet wide. Horizontal cylinder

spacing in excess of 6 feet is desired for working space. From Table D-1.4: Find

horizontal wale with a section modulus of 7.0 and 2 inch diameter cylinders

spaced at 6.5 feet o.c. horizontally. Or, find horizontal wale with a 14.0

section modulus and 3 inch diameter cylinder spaced at 10 feet o.c.

horizontally. Both wales are spaced 4 feet o.c. vertically, 3x12 timber sheeting

is required at close spacing vertically. (See Figure 4 for typical

installation.)

(g) Footnotes, and general notes, for Tables D-1.1, D-1.2, D-1.3, and D-1.4.

(1) For applications other than those listed in the tables, refer to Section

1541.1(c)(2) for use of manufacturer's tabulated data. For trench depths in

excess of 20 feet, refer to Section 1541.1(c)(2) and 1541.1(c)(3).

(2) 2-inch diameter cylinders, at this width, shall have structural steel tube

(3.5 x 3.5 x 0.1875) oversleeves, or structural oversleeves of manufacturer's

specification, extending the full, collapsed length.

(3) Hydraulic cylinders capacities.

(A) 2-inch cylinders shall be a minimum 2-inch inside diameter with a safe

working capacity of not less than 18,000 pounds axial compressive load at

maximum extension. Maximum extension is to include full range of cylinder

extensions as recommended by product manufacturer.

(B) 3-inch cylinders shall be a minimum 3-inch inside diameter with a safe work

capacity of not less than 30,000 pounds axial compressive load at maximum

extension. Maximum extension is to include full range of cylinder extensions as

recommended by product manufacturer.

(4) All spacing indicated is measured center to center.

(5) Vertical shoring rails shall have a minimum section modulus of 0.40 inch.

(6) When vertical shores are used, there must be a minimum of three shores

spaced equally, horizontally, in a group.

(7) Plywood shall be 1.125 inches thick of wood or 0.75 inch thick, 14 ply,

arctic white birch (Finland form). Please note that plywood is not intended as a

structural member, but only for prevention of local raveling (sloughing of the

trench face) between shores.

(8) See Appendix C for timber specifications.

(9) Wales are calculated for simple span conditions.

(10) See Appendix D, Section (d), for basis and limitations of the data.

ALUMINUM HYDRAULIC SHORING TYPICAL INSTALLATIONS

FIGURE NO.1

VERTICAL ALUMINUM HYDRAULIC SHORING (SPOT BRACING)

FIGURE NO.2

VERTICAL ALUMINUM HYDROLIC SHORING (WITH PLYWOOD)

FIGURE NO.3

VERTICAL ALUMINUM HYDRAULIC SHORING (STACKED)

FIGURE NO.4

ALUMINUM HYDRAULIC SHORING WALER SYSTEM (TYPICAL)

TABLE D - 1.1

ALUMINUM HYDRAULIC SHORING

VERTICAL SHORES

FOR SOIL TYPE A

____________________________________________________________________

|

|                  HYDRAULIC CYLINDERS

|__________________________________________________________

|            |          |

DEPTH  |  MAXIMUM   | MAXIMUM  |     WIDTH OF TRENCH (FEET)

OF   | HORIZONTAL | VERTICAL |__________________________________

TRENCH  |  SPACING   | SPACING  |            |          |

|            |          |  UP TO 8   |  OVER 8  |  OVER 12

(FEET) |   (FEET)   |  (FEET)  |            | UP TO 12 | UP TO 15

_________|____________|__________|____________|__________|__________

|            |          |            |          |

OVER   |            |          |            |          |

5     |      8     |          |            |          |

UP TO  |            |          |            |          |

10    |            |          |            |          |

_________|____________|          |            |          |

|            |          |            |          |

OVER   |            |          |            |          |

10    |      8     |     4    |  2 INCH    |  2 INCH  |  3 INCH

UP TO  |            |          | DIAMETER   | DIAMETER | DIAMETER

15    |            |          |            |  NOTE(2) |

_________|____________|          |            |          |

|            |          |            |          |

OVER   |            |          |            |          |

15    |      7     |          |            |          |

UP TO  |            |          |            |          |

20    |            |          |            |          |

_________|____________|__________|____________|__________|__________

|

OVER 20 |              NOTE(1)

_________|__________________________________________________________

Footnotes to tables, and general notes on hydraulic shoring, are found

in Appendix D, Item (g)

Note (1): See Appendix D, Item (g)(1)

Note (2): See Appendix D, Item (g)(2)

TABLE D - 1.2

ALUMINUM HYDRAULIC SHORING

VERTICAL SHORES

FOR SOIL TYPE B

____________________________________________________________________

|

|                  HYDRAULIC CYLINDERS

|__________________________________________________________

|            |          |

DEPTH  |  MAXIMUM   | MAXIMUM  |     WIDTH OF TRENCH (FEET)

OF   | HORIZONTAL | VERTICAL |__________________________________

TRENCH  |  SPACING   | SPACING  |            |          |

|            |          |  UP TO 8   |  OVER 8  |  OVER 12

(FEET) |   (FEET)   |  (FEET)  |            | UP TO 12 | UP TO 15

_________|____________|__________|____________|__________|__________

|            |          |            |          |

OVER   |            |          |            |          |

5     |      8     |          |            |          |

UP TO  |            |          |            |          |

10    |            |          |            |          |

_________|____________|          |            |          |

|            |          |            |          |

OVER   |            |          |            |          |

10    |     6.5    |     4    |  2 INCH    |  2 INCH  |  3 INCH

UP TO  |            |          | DIAMETER   | DIAMETER | DIAMETER

15    |            |          |            |  NOTE(2) |

_________|____________|          |            |          |

|            |          |            |          |

OVER   |            |          |            |          |

15    |     5.5    |          |            |          |

UP TO  |            |          |            |          |

20    |            |          |            |          |

_________|____________|__________|____________|__________|__________

|

OVER 20 |              NOTE(1)

_________|__________________________________________________________

Footnotes to tables, and general notes on hydraulic shoring, are found

in Appendix D, Item (g)

Note (1): See Appendix D, Item (g)(1)

Note (2): See Appendix D, Item (g)(2)

TABLE D - 1.3

ALUMINUM HYDRAULIC SHORING

WALER SYSTEMS

FOR SOIL TYPE B

___________________________________________________________________

|                |                                         |

|     WALES      |          HYDRAULIC CYLINDERS            |

|________________|_________________________________________|

|        |       |                                         |

DEPTH  |VERTICAL|     * |          WIDTH OF TRENCH (FEET)         |

OF   |SPACING |SECTION|_________________________________________|

TRENCH |        |MODULUS|                    |                    |

|        |       |    UP TO 8         | OVER 8 UP TO 12    |

|        |       |____________________|____________________|

|        |       |         |          |         |          |

|        |       |  HORIZ  | CYLINDER |  HORIZ  | CYLINDER |

(FEET) | (FEET) |(IN(3))| SPACING | DIAMETER | SPACING | DIAMETER |

________|________|_______|_________|__________|_________|__________|

|        |       |         |          |         |          |

|        |  3.5  |   8.0   |    2 IN  |   8.0   |   2 IN   |

OVER  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

5    |    4   |       |         |          |         |          |

|        |  7.0  |   9.0   |    2 IN  |   9.0   |   2 IN   |

UP TO  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

10    |        |       |         |          |         |          |

|        | 14.0  |  12.0   |    3 IN  |  12.0   |   3 IN   |

________|________|_______|_________|__________|_________|__________|

|        |       |         |          |         |          |

|        |  3.5  |   6.0   |    2 IN  |   6.0   |   2 IN   |

OVER  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

10   |    4   |       |         |          |         |          |

|        |  7.0  |   8.0   |    3 IN  |   8.0   |   3 IN   |

UP TO  |        |       |         |          |         |          |

|        |_______|_________|__________|_________|__________|

15    |        |       |         |          |         |          |

|        | 14.0  |  10.0   |    3 IN  |  10.0   |   3 IN   |

________|________|_______|_________|__________|_________|__________|

|        |       |         |          |         |          |

|        |  3.5  |   5.5   |    2 IN  |   5.5   |   2 IN   |

OVER  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

15   |    4   |       |         |          |         |          |

|        |  7.0  |   6.0   |    3 IN  |   6.0   |   3 IN   |

UP TO  |        |       |         |          |         |          |

|        |_______|_________|__________|_________|__________|

20   |        |       |         |          |         |          |

|        | 14.0  |   9.0   |    3 IN  |   9.0   |   3 IN   |

________|________|_______|_________|__________|_________|__________|

|                                                          |

OVER   |                                                          |

20    |            NOTE (1)                                      |

________|__________________________________________________________|

Footnotes to tables, and general notes on hydraulic shoring, are found

in Appendix D, Item (g)

Note (1): See Appendix D, Item (g)(1)

Note (2): See Appendix D, Item (g)(2)

*Consult product manufacturer and/or qualified engineer for Section

Modulus of available wales.

**Douglas fir or equivalent with a bending strength not less than 1500 psi.

TABLE D - 1.3

ALUMINUM HYDRAULIC SHORING

WALER SYSTEMS

FOR SOIL TYPE B

[Continued]

____________________________________________________________________

|                |                     |

|     WALES      | HYDRAULIC CYLINDERS | TIMBER UPRIGHTS

|________________|_____________________|____________________

|        |       |                     |

|        |       |  WIDTH OF TRENCH    | MAX. HORIZ SPACING

DEPTH  |VERTICAL|     * |       (FEET)        |   (ON CENTER)

OF   |SPACING |SECTION|_____________________|____________________

TRENCH |        |MODULUS|                     |     |      |

|        |       |  OVER 12 UP TO 15   |     |      |

|        |       |_____________________|     |      |

|        |       |         |           |SOLID| 2 FT | 3 FT

|        |       |  HORIZ  | CYLINDER  |SHEET|      |

(FEET) | (FEET) |(IN(3))| SPACING | DIAMETER  |     |      |

________|________|_______|_________|___________|_____|______|_______

|        |       |         |           |     |      |

|        |  3.5  |   8.0   |    3 IN   |     |      |

OVER  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

5    |    4   |       |         |           |     |      |

|        |  7.0  |   9.0   |    3 IN   | --- | ---  | 3x12

UP TO  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

10    |        |       |         |           |     |      |

|        | 14.0  |  12.0   |    3 IN   |     |      |

________|________|_______|_________|___________|_____|______|______

|        |       |         |           |     |      |

|        |  3.5  |   6.0   |    3 IN   |     |      |

OVER  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

10   |    4   |       |         |           |     |      |

|        |  7.0  |   8.0   |    3 IN   | --- | 3x12 | ---

UP TO  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

15    |        |       |         |           |     |      |

|        | 14.0  |  10.0   |    3 IN   |     |      |

________|________|_______|_________|___________|_____|______|______

|        |       |         |           |     |      |

|        |  3.5  |   5.5   |    3 IN   |     |      |

OVER  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

15   |    4   |       |         |           |     |      |

|        |  7.0  |   6.0   |    3 IN   | 3x12| ---  | ---

UP TO  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

20   |        |       |         |           |     |      |

|        | 14.0  |   9.0   |    3 IN   |     |      |

________|________|_______|_________|___________|_____|______|______

|

OVER   |

20    |            NOTE (1)

________|__________________________________________________________

Footnotes to tables, and general notes on hydraulic shoring, are found

in Appendix D, Item (g)

Note (1): See Appendix D, Item (g)(1)

Note (2): See Appendix D, Item (g)(2)

*Consult product manufacturer and/or qualified engineer for Section

Modulus of available wales.

**Douglas fir or equivalent with a bending strength not less than 1500 psi.

TABLE D - 1.4

ALUMINUM HYDRAULIC SHORING

WALER SYSTEMS

FOR SOIL TYPE C

___________________________________________________________________

|                |                                         |

|     WALES      |          HYDRAULIC CYLINDERS            |

|________________|_________________________________________|

|        |       |                                         |

DEPTH  |VERTICAL|     * |          WIDTH OF TRENCH (FEET)         |

OF   |SPACING |SECTION|_________________________________________|

TRENCH |        |MODULUS|                    |                    |

|        |       |    UP TO 8         | OVER 8 UP TO 12    |

|        |       |____________________|____________________|

|        |       |         |          |         |          |

|        |       |  HORIZ  | CYLINDER |  HORIZ  | CYLINDER |

(FEET) | (FEET) |(IN(3))| SPACING | DIAMETER | SPACING | DIAMETER |

________|________|_______|_________|__________|_________|__________|

|        |       |         |          |         |          |

|        |  3.5  |   6.0   |    2 IN  |   6.0   |   2 IN   |

OVER  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

5    |    4   |       |         |          |         |          |

|        |  7.0  |   6.5   |    2 IN  |   6.5   |   2 IN   |

UP TO  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

10    |        |       |         |          |         |          |

|        | 14.0  |  10.0   |    3 IN  |  10.0   |   3 IN   |

________|________|_______|_________|__________|_________|__________|

|        |       |         |          |         |          |

|        |  3.5  |   4.0   |    2 IN  |   4.0   |   2 IN   |

OVER  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

10   |    4   |       |         |          |         |          |

|        |  7.0  |   5.5   |    3 IN  |   5.5   |   3 IN   |

UP TO  |        |       |         |          |         |          |

|        |_______|_________|__________|_________|__________|

15    |        |       |         |          |         |          |

|        | 14.0  |   8.0   |    3 IN  |   8.0   |   3 IN   |

________|________|_______|_________|__________|_________|__________|

|        |       |         |          |         |          |

|        |  3.5  |   3.5   |    2 IN  |   3.5   |   2 IN   |

OVER  |        |       |         |          |         | NOTE (2) |

|        |_______|_________|__________|_________|__________|

15   |    4   |       |         |          |         |          |

|        |  7.0  |   5.0   |    3 IN  |   5.0   |   3 IN   |

UP TO  |        |       |         |          |         |          |

|        |_______|_________|__________|_________|__________|

20   |        |       |         |          |         |          |

|        | 14.0  |   6.0   |    3 IN  |   6.0   |   3 IN   |

________|________|_______|_________|__________|_________|__________|

|                                                          |

OVER   |                                                          |

20    |            NOTE (1)                                      |

________|__________________________________________________________|

Footnotes to tables, and general notes on hydraulic shoring, are found

in Appendix D, Item (g)

Note (1): See Appendix D, Item (g)(1)

Note (2): See Appendix D, Item (g)(2)

*Consult product manufacturer and/or qualified engineer for Section

Modulus of available wales.

**Douglas fir or equivalent with a bending strength not less than 1500 psi.

TABLE D - 1.4

ALUMINUM HYDRAULIC SHORING

WALER SYSTEMS

FOR SOIL TYPE C

[Continued]

____________________________________________________________________

|                |                     |

|     WALES      | HYDRAULIC CYLINDERS | TIMBER UPRIGHTS

|________________|_____________________|____________________

|        |       |                     |

|        |       |  WIDTH OF TRENCH    | MAX. HORIZ SPACING

DEPTH  |VERTICAL|     * |       (FEET)        |   (ON CENTER)

OF   |SPACING |SECTION|_____________________|____________________

TRENCH |        |MODULUS|                     |     |      |

|        |       |  OVER 12 UP TO 15   |     |      |

|        |       |_____________________|     |      |

|        |       |         |           |SOLID| 2 FT | 3 FT

|        |       |  HORIZ  | CYLINDER  |SHEET|      |

(FEET) | (FEET) |(IN(3))| SPACING | DIAMETER  |     |      |

________|________|_______|_________|___________|_____|______|_______

|        |       |         |           |     |      |

|        |  3.5  |   6.0   |    3 IN   |     |      |

OVER  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

5    |    4   |       |         |           |     |      |

|        |  7.0  |   6.5   |    3 IN   |3x12 | ---  | ---

UP TO  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

10    |        |       |         |           |     |      |

|        | 14.0  |  10.0   |    3 IN   |     |      |

________|________|_______|_________|___________|_____|______|______

|        |       |         |           |     |      |

|        |  3.5  |   4.0   |    3 IN   |     |      |

OVER  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

10   |    4   |       |         |           |     |      |

|        |  7.0  |   5.5   |    3 IN   |3x12 |  --- | ---

UP TO  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

15    |        |       |         |           |     |      |

|        | 14.0  |   8.0   |    3 IN   |     |      |

________|________|_______|_________|___________|_____|______|______

|        |       |         |           |     |      |

|        |  3.5  |   3.5   |    3 IN   |     |      |

OVER  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

15   |    4   |       |         |           |     |      |

|        |  7.0  |   5.0   |    3 IN   | 3x12| ---  | ---

UP TO  |        |       |         |           |     |      |

|        |_______|_________|___________|     |      |

20   |        |       |         |           |     |      |

|        | 14.0  |   6.0   |    3 IN   |     |      |

________|________|_______|_________|___________|_____|______|______

|

OVER   |

20    |            NOTE (1)

________|__________________________________________________________

Footnotes to tables, and general notes on hydraulic shoring, are found

in Appendix D, Item (g)

Note (1): See Appendix D, Item (g)(1)

Note (2): See Appendix D, Item (g)(2)

*Consult product manufacturer and/or qualified engineer for Section

Modulus of available wales.

**Douglas fir or equivalent with a bending strength not less than 1500 psi.

## Shafts.

(a) General.

(1) All wells or shafts over 5 feet in depth into which employees are permitted

to enter shall be retained with lagging, spiling, or casing.

EXCEPTION: Exploration shafts; see Section (d).

(2) The lagging, spiling or casing shall extend at least one foot above ground

level and shall be provided the full depth of the shaft or at least five feet

into solid rock if possible.

(3) All wells, pits, shafts, caissons, etc. shall be barricaded or securely

covered.

(4) Upon completion of exploration and similar operations, temporary wells,

pits, shafts, etc., shall be backfilled.

(b) Small Shafts in Hard Compact Soil.

Two-inch (nominal) cribbing may be used in square shafts not over 4 feet square

in hard compact soil. Each member shall be cut 1/2 way through the width of the

member and dovetailed into position so each member will act as a shore as well

as lagging. Strips shall be nailed in each corner to prevent the boards from

dropping down.

(c) Shafts in Other Than Hard Compact Soil.

(1) A system of lagging supported by braces and corner posts shall be used for

square or rectangular shafts. Corner posts of 4-inch by 4-inch material are

normally acceptable in shafts 4 feet square, or smaller, if they are braced in

each direction with horizontal 4-inch by 4-inch members at intervals not

exceeding 4 feet. Braces and corner posts in larger shafts shall be

correspondingly larger as determined by a civil engineer.

(2) Round shafts shall be completely lagged with 2-inch material which is

supported at intervals not greater than 4 feet by means of adjustable rings of

metal or timber that are designed to resist the collapsing force, or cased in a

manner that provides equivalent protection.

(d) Exploration Shafts. Only a geotechnical specialist shall be permitted to

enter an exploration shaft without lagging, spiling or casing for the purpose of

subsurface investigations under the following conditions:

(1) Initial Inspection. The type of materials and stability characteristics of

the exploration shaft shall be personally observed and recorded by the

geotechnical specialist during the drilling operation. Potentially unsafe

exploration shafts shall not be entered.

(2) Surface Casing. The upper portion of the exploration shaft shall be equipped

with a surface ring-collar to provide casing support of the material within the

upper 4 feet of the exploration shaft. The ring-collar shall extend at least

1-foot above the ground surface.

(3) Gas Tests. Prior to entry into exploration shafts, tests and/or procedures

shall be instituted to assure that the atmosphere within the shaft does not

contain dangerous air contamination or oxygen deficiency. These tests and/or

procedures shall be maintained while working within the shaft to assure that

dangerous air contamination or oxygen deficiency will not occur.

(4) Unstable Local Conditions. The geotechnical specialist shall not descend

below any portion of any exploration shaft where caving or groundwater seepage

is noted or suspected.

(5) Ladder and Cable Descents. A ladder may be used to inspect exploration

shafts 20 feet or less in depth. In deeper exploration shafts, properly

maintained mechanical hoisting devices with a safety factor of at least 6 shall

be provided and used. Such devices shall be under positive control of the

operator being positive powered up and down with fail-safe brakes.

(6) Emergency Standby Employee. An emergency standby employee shall be

positioned at the surface near the exploration shaft opening whenever a

geotechnical specialist is inside the shaft.

(7) Communication. A two-way, electrically-operated communication system shall

be in operation between the standby employee and the geotechnical specialist

whenever boring inspections are being made in exploration shafts over 20 feet in

depth or when ambient noise levels make communication difficult.

(8) Safety Equipment. The following safety equipment shall be used to protect

the geotechnical specialist:

(A) An approved safety harness which will suspend a person upright and that is

securely attached to the hoist cable.

(B) A 12-inch to 18-inch diameter steel cone-shaped headguard/deflector that is

attached to the hoist cable above the harness.

(C) A hoist cable having a minimum diameter of 5/16 inches.

(D) Approved head protection.

(9) Electrical Devices. All electrical devices used within the exploration shaft

by the geotechnical specialist shall be approved for hazardous locations.

(10) Surface Hazards. The storage and use of flammable or other dangerous

materials shall be controlled at the surface to prevent them from entering the

exploration shaft.

## Cofferdams.

(a) If overtopping of the cofferdam by high waters is possible, means shall be

provided for controlled flooding of the work area.

(b) Warning signs for evacuation of employees in case of emergency shall be

developed and posted.

(c) Cofferdam walkways, bridges, or ramps with at least two means of rapid exit,

shall be provided with guardrails as specified in Section 1620.

(d) Cofferdams located close to navigable shipping channels shall be protected

from vessels in transit, where possible.