# Telecommunication Safety

## Application.

(a) This article sets forth safety and health standards that apply to the work

conditions, practices, means, methods, operations, installations and processes

performed at telecommunications centers and at telecommunications field

installations, which are located outdoors or in building spaces used for such

field installations. "Center" work includes the installation, operation,

maintenance, rearrangement, and removal of communications equipment and other associated equipment in telecommunications switching centers. "Field" work includes the installation, operation, maintenance, rearrangement, and removal of conductors and other equipment used for telecommunications service, and of their supporting or containing structures, overhead or underground, on public or private rights of way, including buildings or other structures.

(b) Operations or conditions not specifically covered by this Article are

subject to all the applicable orders contained in the other Safety Orders,

including but not limited to the following: General Industry, Construction and

Electrical Safety Orders.

(c) The requirements of this article shall not apply to existing installations,

provided they are in compliance with the Safety Orders which were in existence

and applicable at the time of installation, and documentation is available to

substantiate this.

(d) All work performed under the provisions of this article shall, unless

otherwise specifically provided for, be performed under the direction of

Qualified Telecommunications Workers or Qualified Electrical Workers trained in

the operations involved.

## Definitions.

Aerial Splicing Platform. A device designed to attach to a cable strand on which

an employee(s) may stand or sit while working aloft.

Aerial Tent. A small tent designed to protect employees in inclement weather

while working on ladders, aerial splicing platforms, or aerial devices.

Barricade. A physical obstruction such as tapes, ropes, cones, or "A" frame-type

wood and/or metal structure intended to warn and limit access to a work area.

Barrier. A physical obstruction which is intended to prevent contact with

energized lines or equipment, or to prevent unauthorized access to work area.

Bond. An electrical connection from one conductive element to another for the

purpose of minimizing potential differences or providing suitable conductivity

for fault current or for mitigation of leakage current and electrolytic action.

Cable. A conductor with insulation, or a stranded conductor with or without

insulation and other coverings (single-conductor cable), or a combination of

conductors insulated from one another (multiple-conductor cable).

Cable Sheath. A protective covering applied to cables.

Circuit. A conductor or system of conductors through which an electric current

is intended to flow. May also be an optical link of glass fiber which provides a

transmission path for pulses of light.

Communication Lines. The conductors and their supporting or containing

structures for telephone, telegraph, railroad signal, data, clock, fire,

police-alarm, community television antenna and other systems which are used for public or private signal or communication service, and which operate at

potentials not exceeding 400 volts to ground or 750 volts between any two points

of the circuit, and the transmitted power of which does not exceed 150 watts.

When communications lines operate at less than 150 volts to ground, no limit is

placed on the capacity of the system. Specifically designed communications

cables may include communication circuits not complying with the preceding

limitations where such circuits are also used incidentally to supply power to

communication equipment.

Conductor. A material, usually in the form of a wire, cable, or bus bar,

suitable for carrying an electric current.

Effectively Grounded. Intentionally connected to earth through a ground

connection or connections of sufficiently low impedance and having sufficient

current-carrying capacity to prevent the build-up of voltages which may result

in undue hazard to employees.

Energized Parts (Live Parts). Parts which are of a potential different from that

of the earth, or some conducting body which serves in place of the earth.

Equipment. A general term which includes materials, fittings, devices,

appliances, fixtures, apparatus, and similar items used as part of, or in

connection with, an electrical supply or communications installation.

Ground (reference). That conductive body, usually earth, to which an electric

potential is referenced.

Ground (as a noun). A conductive connection, whether intentional or accidental,

by which an electric circuit or equipment is connected to reference ground.

Ground (as a verb). The connecting or establishment of a connection, whether by

intention or accident, of an electric current or equipment to reference ground.

Ground Tent. A small tent used to protect employees from inclement weather while working at buried cable pedestal sites or similar locations.

Grounded Conductor. A system or circuit conductor which is intentionally

grounded.

Grounded Systems. A system of conductors in which at least one conductor or

point (usually the middle wire, or the neutral point of transformer or generator

windings) is intentionally grounded, either solidly or through a current-limiting device (not a current-interrupting device).

Grounding Electrode (Ground Electrode). A conductor imbedded in the earth, used for maintaining ground potential on conductors connected to it, and for

dissipating into the earth current conducted to it.

Grounding Electrode Conductor (Grounding Conductor). A conductor used to connect equipment or the grounded circuit of a wiring system to a grounding electrode.

Guy Wire. A tension member used to withstand an otherwise unbalanced force on a pole, crossarm or overhead structure.

Insulated. Separated from other conducting surfaces by a dielectric substance

(including air space) offering a high resistance to the passage of current.

NOTE: When any object is said to be insulated, it is understood to be insulated

in suitable manner for the conditions to which it is subjected. Otherwise, it

is, within the purpose of these orders, uninsulated. Insulating coverings of

conductors are one means of making the conductor insulated.

Insulation (as applied to cable). That which is relied upon to insulate the

conductor from other conductors or conducting parts or from ground.

Joint Use. The sharing of a common facility, such as a manhole, trench or pole,

by two or more different kinds of utilities (e.g., power and

telecommunications).

Ladder Platform. A device designed to facilitate working aloft from an extension

ladder.

Ladder Seat. A removable seat used to facilitate work at an elevated position on

rolling ladders in telecommunication centers.

Manhole. A subsurface enclosure which personnel may enter for the purpose of

installing, operating, and maintaining submersible equipment and/or cable.

Manhole Platform. A platform consisting of separate planks, plywood or other

material which are laid across and secured to steel platform supports. The ends

of the supports are engaged in the manhole cable racks.

Microwave Transmission. The act of communicating or signaling utilizing a

frequency between 1 GHz (gigahertz) and 300 GHz inclusively.

Nominal Voltage. The nominal voltage of a system or circuit is the value

assigned to a system or circuit of a given voltage class for the purpose of

convenient designation. The actual voltage may vary above or below this value.

Optical Link. Glass strands encased in flexible cables used to transmit pulses

of light from one point to another.

Pole Balcony or Seat. A balcony or seat used as a support for workers at

pole-mounted equipment or terminal boxes. A typical device consists of a bolted

assembly of steel details and a wooden platform. Steel braces run from the pole

to the underside of the balcony.

Pole Platform. A platform intended for use by a worker in an elevated position

adjacent to a pole.

Qualified Person. A person designated by the company who by reason of training

and experience has demonstrated the ability to safely perform assigned duties.

Qualified Telecommunications Worker. A worker who, through related training

and/or on the job experience, is familiar with the techniques and hazards of

telecommunications work and the equipment used in the specific operations

involved; and has demonstrated the ability to safely perform assigned duties,

and, where required, is properly licensed in accordance with federal, state, or

local laws and regulations.

Strand. Wires laid parallel and twisted used to support wires, cables and

equipment; sometimes called aerial strand, supporting strand, suspension strand,

cable strand or messenger.

Suitable.

(A) Function. Capable of performing with safety the particular function

specified in these Orders, or

(B) Equipment. Equipment or devices intended for the anticipated exposure which

by way of design, strength and quality will eliminate, preclude or mitigate a

particular hazard, and

(C) Use. The conditions, practices, means, operations or processes used which

will eliminate, preclude or mitigate a particular hazard.

Telecommunications. A science dealing with the provisions of Communication

Service.

Telecommunication Center. An installation of communication equipment under the exclusive control of an organization providing telecommunications service, that is located outdoors or in a vault, chamber, or a building space used primarily

for such installations.

NOTE: Telecommunication centers are facilities established, equipped and

arranged in accordance with engineered plans for the purpose of providing

telecommunications service. They may be located on premises owned or leased by the organization providing telecommunication service, or on the premises owned or leased by others. This definition includes switch rooms (whether

electromechanical, electronic, or computer controlled), terminal rooms, power

rooms, repeater rooms, transmitter and receiver rooms, switchboard operating

rooms, cable vaults, and miscellaneous communications equipment rooms.

Simulation rooms of telecommunication centers for training or developmental

purposes are also included.

Telecommunication Line Truck. A truck used to transport personnel, tools, and

material, and to serve as a traveling workshop for telecommunication

installation and maintenance work. It is usually equipped with auxiliary

equipment for setting poles, digging holes, and elevating material or personnel.

Telecommunication Service. The furnishing of a capability to signal or

communicate at a distance by means such as telephone, telegraph, police and fire alarm, community antenna television, or similar system, using wire, conventional cable, coaxial cable, wave guides, microwave and radio transmission, or other similar means.

Vault. An enclosure above or below ground which personnel may enter and which is used for the purpose of installing, operating, and/or maintaining equipment and/or cable which need not be of submersible design.

Vault, Unvented. A vault in which the only openings are access openings.

Vault, Vented. A vault with provision for air changes using exhaust flue

stack(s) and air intake(s), which operate on differentials of pressure and/or

temperature resulting in air flow throughout the vault.

## General.

(a) Illumination. Illumination shall be provided as needed to perform the work

safely.

(1) Telecommunications Centers. Lighting in telecommunication centers shall be

provided in an adequate amount such that continuing work operations, routine

observations, and the passage of employees can be carried out in a safe and

healthful manner.

Additional lighting shall be provided as required for specific tasks.

(2) Field Work. Whenever natural light is inadequate, suitable artificial

illumination shall be provided.

(b) Working Surfaces. Guardrails and toeboards may be omitted on distribution

frame mezzanine platforms to permit access to equipment. This exemption applies only on the side or sides of the platform facing the frames and only on those portions of the platform adjacent to equipment frames.

(c) Working Spaces, Aisles. Working spaces provided by "maintenance aisles," or  "wiring aisles," between equipment frames lineups in telecommunications centers shall not be considered as parts of egress of the General Industry Safety Orders.

(d) Special Doors. When blastproof or power-actuated doors are installed in

specially designed hardsite security buildings and spaces, they shall be

designed and installed so that they can be used as a means of egress in

emergencies.

(e) Medical and First Aid. First-aid supplies recommended by a consulting

physician and shall be maintained and shall be easily accessible.

(f) Hazardous Substances. All work involving hazardous substances shall comply

with Group 16 of the General Industry Safety Orders. Highway Mobile Vehicles and Trailers may be equipped to carry more than one LP gas container provided all provisions of the Unfired Pressure Vessel Safety Orders are complied with.

(g) Work Space. Suitable means shall be provided to permit employees to safely

perform work on telecommunications equipment.

(h) Approach Distances to Exposed Energized Overhead Power Lines or Parts. The company shall require that no employee approaches or takes any conductive object closer to any electrically energized overhead power lines or parts than prescribed in Table TC-1, unless:

(1) The employee is insulated or guarded from the energized parts (insulating

gloves rated for the voltage involved shall be considered adequate insulation

for the employee from the energized parts for voltage not to exceed 7,500

volts); or

(2) The energized parts are insulated or guarded from the employee and any other conductive object at a different potential; or

(3) The power conductors and equipment are de-energized and grounded.

Table TC-1

Approach Distances to Exposed

Energized Overhead Power Lines and Parts

by Qualified Telecommunication WorkersVolage Range

(Phase to Phase, RMS)Approach Distance

(Inches)

300 V and less						12

Over 300 V, not over 600 V				18

Over 600 V, not over 2kV				24

Over 2kV, not over 15kV				36

Over 15kV, not voer 37kV				42

Over 37kV, not voer 87.5k				48

Over 87.5kV, not over 121kV				54

Over 121kV, not over 140kV				Avoid Contact

(i) Inclement Weather. Work from structures shall be discontinued when adverse

weather such as high winds, ice on structures, or progress of an electrical

storm in the immediate vicinity, makes the work unsafe.

## Training.

(a) Companies shall provide training in the various precautions and safe

practices relating to employees' job assignment(s) as described in this article

and other applicable orders, and shall require that employees do not engage in

these activities until they have received proper training. However, where the

company can demonstrate that an employee is already trained in the precautions

and safe practices required herein prior to employment, additional training need

not be provided. Where training is required, it shall consist of on-the-job or

classroom-type training or a combination of both. The company shall certify

that employees have been trained by preparing a certification record which

includes the identity of the person trained, the signature of the company or

the person who conducted the training, and the date the training was completed.

The certification record shall be prepared at the completion of the training

maintained for the duration of the employee's assignment. The certification

record shall be made available upon request to the Division. Such training

shall, where appropriate, include the following subjects:

(1) Recognition and avoidance of dangers relating to encounters with harmful

substances, and animal, insect, or plant life.

(2) Procedures to be followed in emergency situations.

(3) First-aid training, including instruction in artificial respiration and/or

cardiopulmonary resuscitation and refresher course as needed to keep training

updated.

(4) Work area protection and traffic control.

## Employee Protection in Public Areas.

(a) Before work is begun in the vicinity of vehicular or pedestrian traffic

which may endanger employees, warning signs and/or flags or other traffic

control devices shall be placed conspicuously to alert and channel approaching

traffic. Where further protection is needed, barriers shall be utilized. At

night, warning lights shall be prominently displayed and excavated areas shall

be enclosed with protective barricades.

(b) If work exposes energized or moving parts that are normally protected,

danger signs shall be displayed and barricades erected, as necessary, to warn

other personnel in the area.

(c) The company shall require that an employee finding any crossed or fallen

wires which create or may create a hazardous situation at the work area:

(1) Remains on guard or adopts other adequate means to warn other employees of the danger; and

(2) Has the proper authority notified at the earliest practical moment.

(d) Company shall require employees exposed to vehicular traffic hazards

outside the protected area to comply with the provisions specified in Construction Safety.

## Tools and Personal Protective Equipment.

(a) Personal protective equipment, protective devices and special tools needed

for the work of employees shall be provided and the company shall require that

they are used by employees. Before each day's use the company shall ensure that these personal protective devices, tools, and equipment are carefully inspected by a qualified person to ascertain that they are in good condition.

(b) Defective tool and equipment shall be removed from service.

(c) Telecommunication linemen's body belts, safety straps and lanyards shall

meet the requirements contained in the High Voltage Electrical Safety Orders.

## Rubber Insulating Equipment.

Rubber insulating equipment shall comply with the provisions of the High-Voltage Electrical Safety Orders.

## Pole Climbers.

(a) Pole climbers shall not be used if the gaffs are less than 1 1/4 inches in

length as measured on the underside of the gaff. The gaffs of pole climbers

shall be covered with safety caps or equivalent protection when not being used

for their intended use.

(b) The company shall require that pole climbers are visually inspected by a

qualified person for the following conditions: Fractured or cracked gaffs or leg

irons, loose or dull gaffs, broken straps or buckles. If any of these conditions

exist, the defect shall be corrected before the climbers are used.

(c) Pole climbers shall be inspected as required in this section before each

day's use and a gaff cut-out test performed at least weekly when in use.

(d) Pole climbers shall not be used except on wood poles.

Poles, Towers and Ladders.

(a) The spacing between permanent and detachable steps installed on poles and

towers shall be not more than 18 inches (36 inches on any one side). Spacing

shall be uniform except where working, standing or access steps are required.

The lowest permanent step shall be not less than 7 feet 6 inches from the ground

line. Permanent steps for poles and towers shall not unnecessarily obstruct the

climbing space and shall have a minimum diameter of 5/8-inch and a minimum clear tread width of 4 1/2 inches. The lowest detachable step shall be no more than 24 inches above ground level. Detachable steps shall be properly secured when in use.

(b) Pole Steps. Wood poles (such as poles with pole and/or strand mounted

terminals) which are expected to be frequently climbed for maintenance or

operating purposes shall be stepped in accordance with the regulations of the

[State] Public Utilities Commission.

(c) The company shall require that no employee nor any material or equipment

will be supported on any portion of a ladder unless it is first determined by

inspection by a qualified person that such ladder is adequately strong, in good

condition, and properly secured in place.

(d) Portable metal ladders shall not be used in telecommunications work, except

in vaults and manholes.

(e) Portable wood ladders intended for general use shall not be painted but may be coated with a translucent nonconductive coating. Portable wood ladders shall not be longitudinally reinforced with metal.

(f) Portable wood ladders that are not being carried on vehicles and are not in

active use shall be stored under cover.

(g) The applicable provisions of the General Industry Safety Orders shall apply

to rolling ladders used in telecommunications centers, except that existing

ladders may have a minimum inside width, between the side rails, of at least 8

inches.

(h) When in use, rolling ladders shall be secured to prevent inadvertent

displacement.

(i) Climbing ladders or stairways on scaffolds used for access and egress shall

be affixed or built into the scaffold by proper design and engineering, and

shall be so located that their use will not disturb the stability of the scaffold.

(1) If a portable ladder is affixed to the scaffold, it shall be securely

attached and shall have rungs meeting the spacing and clearances requirements.

(2) If a fixed ladder is installed on the scaffold it shall comply with the General Industry Safety Orders.

(3) Stairways shall comply with the General Industry Safety Orders.

(4) Horizontal end braces may also be designed and used as a climbing device

provided that the steps are:

(A) Parallel and level.

(B) Uniformly spaced throughout the climb.

(C) Not less than 16 inches in width.

(D) Not less than 12 inches nominal nor more than 16 inches nominal apart on center.

(E) Provided with sufficient clearance to provide a good handhold and foot

space.

(5) Those surfaces of rungs and steps designed for use in ascending and

descending shall be corrugated, serrated, knurled, dimpled, or coated with

skid-resistant material.

(j) When a ladder is supported by an aerial strand, and ladder hooks or other

supports are not being used, the ladder shall be extended at least 2 feet above

the strand and shall be secured to it (e.g. lashed or held by a safety strap

around the strand and ladder side rail). When a ladder is supported by a pole,

it shall be securely lashed to the pole unless the ladder is specifically

designed to prevent movement when used in this application.

## Other Tools and Personal Protective Equipment.

(a) Tent Heaters. Flame-type heaters shall not be used within ground tents or on

platforms within aerial tents unless:

(1) The tent covers are constructed of fire-resistant materials, and

(2) Adequate ventilation is provided to maintain safe oxygen levels and avoid

harmful build-up of combustion products and combustible gases.

(b) Torches. Torches may be used on aerial splicing platforms or in buckets

enclosed by tents provided the tent material is constructed of fire-resistant

material and the torch is turned off when not in actual use. Aerial tents shall

be adequately ventilated while the torch is in operation.

(c) Soldering Devices. Electrical soldering devices shall be grounded. Defective

soldering devices shall be immediately removed from service.

EXCEPTION: 1: Soldering devices used exclusively for work on live circuits. Such devices shall be tested for current leakage not less than each 3 months.

EXCEPTION: 2: Soldering devices supplied through an isolating transformer with

an ungrounded secondary of not over 50 volts.

EXCEPTION: 3: Soldering devices protected by a system of double insulation, or

its equivalent. Where such a system is employed, the equipment shall be

distinctively marked.

(d) Lead Work. All lead work shall comply with the General Industry

Safety Orders. The wiping of lead joints using melted solder, gas fueled

torches, soldering irons or other appropriate heating devices, and the soldering

of wires or other electrical connections do not constitute the welding, cutting

and brazing described in Group 10 of the General Industry Safety Orders. When

operated from commercial power the metal housing of electric solder pots shall

be grounded. The company shall require that wiping gloves or cloths and eye protection are used in lead wiping operations. A drip pan to catch hot lead drippings shall also be provided and used.

## Vehicle-Mounted Material Handling Devices                                                        and Other Mechanical Equipment  (General).

(a) The company shall require that visual inspections are made of the equipment

by a qualified person each day the equipment is to be used to ascertain that it

is in good condition.

(b) The company shall require that tests shall be made at the beginning of each

shift by a qualified person to ensure the vehicle brakes and operating systems

are in proper working condition.

(c) All rubber-tired, self-propelled scrapers, rubber-tired front-end loaders,

rubber-tired dozers, agricultural and industrial tractors, crawler tractors,

crawler-type loaders, and motor graders, with or without attachments, that are

used in telecommunications work shall have roll-over protective structures, when

required, in accordance with Article 25 of the General Industry Safety Orders or

Section 1596 of the Construction Safety Orders as applicable.

(d) Vehicle-mounted elevating and rotating work platform devices shall not be

operated with any conductive part of the equipment closer to exposed energized

power lines than the clearances set forth in Table TC-1 of this Article.

## Hoisting Equipment.

(a) Hoisting equipment shall not be operated with any conductive part of the

equipment closer to exposed energized high-voltage conductors and equipment than the clearances set forth in Table TC-1 of this Article.

(b) When hoisting equipment is used to handle poles near energized high-voltage

conductors and equipment, these operations shall comply with the requirements

contained in the General Section (h) and Overhead Lines (i).

(c) Moving parts of equipment and machinery carried on or mounted on

telecommunications vehicles shall be guarded, operated, and maintained in

accordance with applicable provisions of the General Industry Safety Orders.

## Materials Handling And Storage

(a) Poles. When working with poles in piles or stacks, work shall be performed

from the ends of the poles as much as possible, and precautions shall be taken

for the safety of employees at the outer end of the pole. During pole hauling

operations, all loads shall be secured to prevent displacement. Lights,

reflectors and/or flags shall be displayed on the end and sides of the load as

necessary. In the case of hoisting machinery equipped with a positive stop load-holding device, it shall be permissible for the operator to leave the controls (while a load is suspended) for the sole purpose of assisting in positioning the load prior to landing it. Prior to and during the unloading steel, poles, crossarms, and similar material, the load shall be thoroughly examined to ascertain that the load has not shifted, that binders or stakes have

not broken, and that the load is not otherwise hazardous to employees. Employees shall not stand on top or in the potential path of an unsecured load while unloading poles from pole dollies or utility trailers.

(b) Cable Reels. Cable reels shall be chocked or otherwise restrained when there

is a possibility of inadvertent movement.

## Cable Fault Locating and Testing.

(a) Employees involved in using high voltages to locate trouble or test cables

shall be instructed in the precautions necessary for their own safety, and the

safety of other employees.

(b) Before the voltage is applied, cable conductors shall be isolated. Employees

shall be warned, by such techniques as briefing and tagging at all affected

locations, to stay clear while the voltage is applied.

## Grounding for Employee Protection--Pole Lines.

(a) Power Conductors. Electric power conductors and equipment shall be

considered as energized unless the employee can visually determine that they are bonded to one of the grounds listed in Subsection (d) of this Section.

(b) Nonworking Open Wire. Nonworking open wire communications lines shall be

bonded to one of the grounds listed in Subsection (d) of this Section.

(c) Vertical Power Conduit, Power Ground Wires and Street Light Fixtures.

(1) Metal power conduit, exposed vertical power ground wires, and street light

fixtures which are below communications attachments or less than 20 inches above these attachments, shall be considered energized and shall be tested for voltage unless the employee can visually determine that they are bonded to the

communications suspension strand or cable sheath.

(2) If no hazardous voltage is shown by the voltage test, a temporary bond shall

be placed between such street light fixture, exposed vertical power grounding

conductor, or metallic power conduit and the communications cable strand.

Temporary bonds used for this purpose shall have sufficient conductivity to

carry at least 500 amperes for a period of one second without fusing.

(d) Suitable Protective Grounding. Acceptable grounds for protective grounding

are as follows:

(1) A vertical ground wire which has been tested, found safe, and is connected

to a power system multigrounded neutral or the grounded neutral of a power

secondary system where there are at least three services connected;

(2) Communications cable sheath or shield and its supporting strand where the

sheath or shield is:

(A) Bonded to an underground or buried cable which is connected to a central

office ground, or

(B) Bonded to an underground metallic piping system, or,

(C) Bonded to a power system multigrounded neutral or grounded neutral of a

power secondary system which has at least three services connected;

(3) Guys which are bonded to the grounds specified in Subsection (d)(1) and (2)

of this Section and which have continuity uninterrupted by an insulator; and

(4) If all of the preceding grounds are not available, arrays of driven ground

rods where the resultant resistance to ground will be low enough to eliminate

danger to personnel or permit prompt operation of protective devices.

(e) Attaching and Removing Temporary Bonds. When attaching grounds (bonds), the first attachment shall be made to the protective ground. When removing bonds, the connection to the line or equipment shall be removed first. Insulating

gloves shall be worn during these operations.

(f) Temporary Grounding of Suspension Strand.

(1) The suspension strand shall be grounded to the existing grounds listed in

Subsection (d) of this Section when being placed on jointly used poles and while

performing work during thunderstorms.

(2) Where unsupported power crossings are encountered, the strand shall be

bonded to an existing ground listed in Subsection (d) of this Section as close

as possible to the crossing. This bonding is not required where crossings are

made on a common crossing pole unless there is an upward change in grade at the pole. (Tie-down stress on the strand.) In this case, the bonding shall be at an

adjacent pole.

(3) Where roller-type bonds are used, they shall be restrained so as to avoid

stressing the electrical connections.

(4) Bonds between the suspension strand and the existing ground shall be at

least No. 6 AWG copper.

(5) Temporary bonds shall be left in place until the strand has been tensioned,

dead-ended, and permanently grounded.

EXCEPTION: The installation of insulated strand.

(g) Antenna Work-Radio Transmitting Stations 3-30 MHZ.

(1) Prior to grounding a radio transmitting station antenna, the company shall

ensure that the person in charge:

(A) Prepares and signs a danger tag,

(B) Requests the transmitting technician to shut down the transmitter and to

ground the antenna with its grounding switch,

(C) Is notified by the transmitting technician that the transmitter has been

shut down, and

(D) Tags the antenna ground switch personally in the presence of the

transmitting technician after the antenna has been grounded by the transmitting

technician.

(2) Power shall not be applied to the antenna, nor shall the grounding switch be

opened under any circumstances while the tag is affixed.

(A) Where no grounding switches are provided, grounding sticks shall be used,

one on each side of line, and tags shall be placed on the grounding sticks,

antenna switch, or plate power switch in a conspicuous place.

(B) When necessary to further reduce excessive radio frequency pickup, ground

sticks or short circuits shall be placed directly on the transmission lines near

the transmitter in addition to the regular grounding switches.

(C) In other cases, the antenna lines may be disconnected from ground and the

transmitter to reduce pickup at the point in the field.

(3) All radio frequency line wires shall be tested for pickup with an insulated

probe before they are handled either with bare hands or with metal tools.

(4) The company shall require that the transmitting technician warn the riggers

about adjacent lines which are, or may become, energized.

(5) The company shall require that when antenna work has been completed, the

person in charge of the job returns to the transmitter, notifies the

transmitting technician in charge that work has been completed, and personally

removes the tag(s) previously attached.

## Overhead Lines.

(a) Handling Suspension Strand.

(1) The company shall ensure that when handling cable suspension strand which

is being installed on poles carrying exposed energized power conductors,

employees shall wear insulating gloves and shall avoid body contact with the

strand until after it has been tensioned, dead-ended and permanently grounded.

(2) The strand shall be restrained against inadvertent upward movement during

installation:

(A) On joint-use poles, where there is an upward change in grade at the pole,

and

(B) On non-joint-use poles, where the line crosses under energized power

conductors.

(b) Need for Testing Wood Poles.

Unless temporary guys or braces are attached, the following poles shall be

tested in accordance with Subsection (c) of this Section and determined to be

safe before employees are permitted to climb them:

(1) Dead-end poles, except properly braced or guyed "Y" or "T" cable junction

poles,

(2) Straight line poles which are not storm guyed and where adjacent span

lengths exceed 165 feet.

(3) Poles at which there is a downward change in grade and which are not guyed

or braced corner poles or cable junction poles.

(4) Poles which support only telephone drop wire, and

(5) Poles which carry less than ten communication line wires. On joint-use

poles, one power line wire shall be considered as two communication wires for

purposes of this subsection.

(6) Poles at which the cable strand or guy tension is to be changed.

(c) Pole Inspection and Tests.

(1) Companies shall establish an effective written pole inspection and testing

program and all involved employees shall be trained therein.

(2) Prior to climbing erected poles or other elevated structures intended to

support overhead lines or equipment, a qualified person(s) shall conduct

inspections and tests as required by Section 8615(b) to assure that such poles

or structures are in safe condition for the work to be performed.

(3) Where poles or structures are determined to be unsafe for climbing, they

shall be marked and not be climbed until made safe by guying, bracing, or other

adequate means.

(d) Test Requirements for Cable Suspension Strand.

(1) Before attaching a splicing platform to a cable suspension strand, the

strand shall be tested and determined to have strength sufficient to support the

weight of the platform and the employee. Where the strand crosses above power

conductors or railroad tracks it may not be tested but shall be inspected in

accordance with Subsection (f) of this Section.

(2) The following method or an equivalent method shall be used for testing the

strength of the strand: A rope, at least 3/8-inch in diameter, shall be thrown

over the strand. On joint lines, the rope shall be passed over the strand using

tree pruner handles or a wire raising tool. If two employees are present, both

shall grip the double rope and slowly transfer their entire weight to the rope

and attempt to raise themselves off the ground. If only one employee is present,

one end of the rope which has been passed over the strand shall be tied to the

bumper of the truck, or other equally secure anchorage. The employee then shall

grasp the other end of the rope and attempt to rise off the ground.

(e) Inspection of Strand. Where strand passes over electric power conductors or

railroad tracks, it shall be inspected from an elevated working position at each

pole supporting the span in question. The strand shall not be used to support

any splicing platform, scaffold or cable car, if any of the following conditions

exist:

(1) Corrosion so that no galvanizing can be detected,

(2) One or more wires of the strand are broken,

(3) Worn spots, or

(4) Burn marks such as those caused by contact with electric power conductors.

(f)(1) Elevated Work Platforms. Unless standard railings meeting the

requirements of Section 3209 of the General Industry Safety Orders are provided, suitable fall-arresting devices such as safety straps and body belts shall be used while working on elevated work platforms including aerial splicing

platforms, pole platforms, ladder platforms, pole balconies and pole seats.

(2) Ladder Platform. A ladder platform shall consist of a supporting surface not

less than 9 inches by 18 inches. The rear edge of the platform and the support

member shall be equipped to lock the platform to the ladder rungs.

(3) Pole Platform. A pole platform shall consist of a supporting surface not

less than 10 inches by 40 inches equipped at one end with a hinged chain binder

for securing the platform to a pole. A brace shall be provided from the pole to

the underside of the platform.

(4) Splicing Platform. An aerial splicing platform shall consist of a supporting

surface approximately 3 feet by 4 feet furnished with fiber or synthetic ropes

for suspending the platform, detachable guy ropes for anchoring it, and a device

for raising and lowering it with a handline.

(g) When work is performed where the employee's feet are at a position more than 4 feet above ground on poles, towers or similar structures, the company shall require the employee use body belts and/or safety belts and safety straps and/or lanyards.

(h) Installing and Removing Wire and Cable. Before installing or removing wire

or cable, the pole or structure shall be guyed, braced, or otherwise supported,

as necessary, to prevent failure of the pole or structure.

(i) Handling Poles Near Energized High-Voltage Conductors and Equipment. Except qualified electrical workers complying with Article 36 of High-Voltage

Electrical Safety Orders:

(1) Joint-use poles shall not be set, moved, or removed where the nominal

voltage of open electrical power conductors exceeds 34.5kV phase to phase (20kV to ground).

(2) In joint lines where the power voltage is greater than 600 volts but less

than 34.5kV phase to phase (20kV to ground), poles being placed, moved, or

removed shall be insulated with either a rubber insulating blanket, a fiberglass

box guide, or equivalent protective equipment. Work procedures shall be

developed and employed to minimize the possibility of the pole and the

protective equipment from contacting the power conductors. The guard or

insulating material used to protect the pole shall meet the appropriate 3-minute

proof test voltage requirements contained in the ASTM-D-1048-1981, which are

hereby incorporated by reference.

(3) Where poles are being removed, insulation of the pole is not required if the

pole is cut off 2 feet or more below the lowest power conductor and also cut off

near the ground line.

(4) Suitable insulating gloves shall be worn when handling the pole with either

hands or tools, when there exists a possibility that the pole may contact an

energized high-voltage conductor.

(5) During pole handling operations, employees standing on the ground shall not

be permitted to step onto or contact equipment or machinery while working near

energized high-voltage conductors or equipment unless the employees are using

suitable protective equipment for the voltages involved.

(j) Metal Tapes and Ropes.

(1) Metal measuring tapes, metal measuring ropes, or tapes containing conductive strands shall not be used when working near exposed energized parts.

(2) Where it is necessary to measure clearances from energized parts, only

nonconductive devices shall be used.

## Underground Lines.

The provisions of this section apply to the guarding of manholes and street

openings, and to the ventilation and testing for gas in manholes and unvented

vaults, where telecommunications field work is performed on or with underground

lines.

(a) Guarding Manholes and Street Openings.

(1) When covers of manholes or vaults are removed, the opening shall be promptly guarded by a railing, temporary cover, or other suitable temporary barrier which is appropriate to prevent an accidental fall through the opening and to protect employees working in the manhole from foreign objects entering the manhole.

(2) While work is being performed in the manhole, a person with basic first-aid

training shall be immediately available to render assistance if there is cause

for believing that a hazard exists. Examples of manhole worksite hazards which shall be considered to constitute a hazard include, but are not limited to:

(A) Manhole worksites where hazards are created by traffic patterns that cannot

be corrected by provisions of Section 8604(a) and Section 8616(a)(1).

(B) Manhole worksites that are subject to unusual water hazards that cannot be

abated by conventional means.

(C) Manhole worksites that are occupied jointly with power utilities as

described in Subsection (c) of this Section.

(D) Where combustible or explosive gas vapors have been initially detected at an

explosive level.

(E) Where toxic or otherwise hazardous substances are used in such quantities

that the work operations or inadvertent spills would create unsafe airborne

concentrations or other bodily injury exposures.

(F) Where open flame torches are used in the work procedure.

(G) Where hazardous substances:--i.e. pesticides, herbicides, etc. are

encountered.

(H) When employees are required to perform extended night work in manholes,

except where constant voice communication is maintained with other person(s) who can render ready assistance. This shall not prohibit performing emergency

trouble work without assistance providing the work can be performed safely.

(b) Requirements Prior to Entering Manholes and Unvented Vaults.

(1) Before an employee enters a manhole or unvented vault, the following steps

shall be taken:

(A) The company shall determine that a confined space condition as defined in

General Industry Safety Orders Article 108 is non-existent. If a confined space

condition exists, the requirements of Article 108 shall be followed. Otherwise,

the requirements set forth herein shall be followed.

(B) The internal atmosphere shall be tested for combustible gas and, except when continuous forced ventilation is provided, the atmosphere shall also be tested for oxygen deficiency.

(C) When unsafe conditions are detected by testing or other means, the work area shall be ventilated by mechanical means and otherwise made safe before entry.

(2) An adequate continuous supply of air shall be provided while work is

performed in unvented vaults and manholes under any of the following conditions:

(A) Where combustible or explosive gas vapors have been initially detected and

subsequently reduced to a safe level by ventilation,

(B) Where organic solvents are used in the work procedure,

(C) Where open flame torches are used in the work procedure,

(D) Where the manhole or vault is located in that portion of a public

right-of-way open to vehicular traffic and/or exposed to a seepage of gas or

gases, or

(E) Where a toxic gas or oxygen deficiency is found.

EXCEPTION: Work in central office cable vaults that are adequately ventilated.

(c) Manhole Platform. A manhole platform shall consist of nominal 2-inch by

12-inch lumber of a grade no less than that of a "Structural Plank" as defined

(see Lumber) in Section 1504 "Definition" of the Construction Safety Orders, or

other nonconductive material of equal strength.

(d) Joint Power and Telecommunication Manholes. While work is being performed in a manhole occupied jointly by an electric utility and a telecommunication utility, an employee with basic first-aid training shall be available in the immediate vicinity to render emergency assistance as may be required. The employee whose presence is required in the immediate vicinity for the purposes of rendering emergency assistance is not to be precluded from occasionally entering a manhole to provide assistance other than in an emergency.

(e) Ladders. Metal ladders having non-skid rungs, wooden ladders or portable

reinforced plastic ladders shall be used to enter and exit manholes exceeding 4

feet in depth.

(f) Flames. When open flames are used in manholes, the following precautions

shall be taken to protect against the accumulation of combustible gas:

(1) A test for combustible gas shall be made immediately before using the open

flame device, and at least once per hour while using the device; and

(2) A fuel tank (e.g., acetylene) may not be in the manhole unless in actual use.

## Eye Protection.

(a) Microwave. Companies shall require that employees do not look into an open

waveguide which is connected to an energized source of microwave radiation.

(b) Fiber Optics. Companies shall require that employees do not look:

(1) Into the beam of an operating laser diode beam, or

(2) Into the end of an optical fiber emitting sufficient energy to cause damage

to the eye.

## Microwave Transmission.

(a) Hazardous Area. Accessible areas associated with microwave communication systems where the electromagnetic radiation level exceeds the radiation exposure limits given in Section 5085 of the General Industry Safety Orders shall be posted as described in that Section. The lower half of the warning symbol shall include the following:

Radiation in this area may exceed hazard limitations and special precautions are

required. Obtain specific instruction before entering.

(b) Protective Measures. When an employee works in an area where the

electromagnetic radiation exceeds the radiation protection guide, the company

shall institute measures that ensure that the employee's exposure is not greater

than that permitted by the radiation guide. Such measures shall include, but not

be limited to, those of an administrative or engineering nature or those

involving personal protective equipment.

## Working Alone

Working Alone: In the event that a worker is working alone, the supervisor will ensure that periodic contact is made with the employee on a regular basis, as scheduled by the job hazard assessment. If the nature of the work were hazardous, then a shorter time frame would be set up. Contact will be made via cell phones or radios.

### Application Of Working Alone

[Company_Name] from time to time may require that an employee be on a job site working alone. Working alone is a potential hazard for any worker or employee not matter how experienced they are in their field of work. During this time the employee who is working alone does not have any other assistance or if help is  not  readily  available  in the case of an  emergency  or  the worker is injured or ill, the following will apply as the policy and procedure for [Company_Name]:

### Hazard Assessment

To assess this hazard [Company_Name] will review records and past incidents, and identify measures or actions needed to correct any hazards.

### Precautions Required

#### Effective Communication

[Company_Name] will provide effective radio, telephone or other electronic communication  between  a  worker  who  works  alone  and  persons  capable of assisting the worker in an emergency or if the worker is injured or ill.

#### Alternate Forms Of Communication

If effective electronic communication is not practicable or readily available at the work

site, the [Company_Name] will ensure that :

(a) the employer or another competent worker visits the worker, or*

(b) the worker contacts the employer or another competent worker*.

* See the below subsection.

#### Visit & Contact Time Intervals

*The  visits  or  contact  under  the above section must  be  at  intervals  of  time

appropriate to the nature of the hazards associated with the worker’s work.

## Rescue and Emergency Services

### Overview

[Company_Name] has develop and implement procedures for summoning rescue and emergency services, for rescuing climbers from elevated heights on poles and towers, for providing necessary emergency services to rescued employees, and for preventing unauthorized personnel from attempting a rescue.

[Company_Name] has certified that the rescue and emergency services have been trained, certified and will:

- Evaluate a prospective rescuer's ability to respond to a rescue summons in a timely manner, considering the hazard(s) identified;

- Evaluate a prospective rescue service's ability, in terms of proficiency with rescue-related tasks and equipment, to function appropriately while rescuing climbers from the particular elevated heights on pole and towers identified;

- Select a rescue team or service from those evaluated that:

  - Has the capability to reach the victim(s) within a time frame that is appropriate for securing the safety of the climber;

  - Is equipped for and proficient in performing the needed rescue services;

  - Inform each rescue team or service of the hazards they may confront when called on to perform rescue at the site; and

  - Provide the rescue team or service selected with access to all elevated heights on pole and towers from which rescue may be necessary so that the rescue service can develop appropriate rescue plans and practice rescue operations.

## Tower Climbers

Each and every day prior to climbing a tower [Company_Name] will have a pre-climb meeting and equipment inspection. [Company_Name] will adhere to our ‘Fall Protection’ chapter in this manual as well as the Tower Climbers items detailed in this chapter and section.

(a) Tower climbers shall be covered with safety caps and gear or equivalent protection when not being used for their intended use.

(b) The company shall require that tower climbers are visually inspected by a

qualified person for the proper climbing equipment that is in good condition and does not appear to be faulty or excessively worn. If any of these conditions exist, the defect shall be corrected before the climbers are allowed to climb.

(c) Tower climbers shall be inspected as required in this section before each

day's use or climb.

(d) Tower climbers are required to adhere to a 100% tie-off policy at all times, NO EXCEPTIONS. This will save your life!!! [Company_Name] has a 100% tie-off policy.